SECTION H: REFERENCE DOCUMENTS

CONTENTS

Item	Page

REFERENCE DOCUMENTS	2

H.1	GENERAL	2

H.2	REFERENCES TO DESIGN GUIDELINES AND OTHER INFORMATION DOCUMENTS TO BE USED FOR WORK FOR THE NATIONAL DEPARTMENT OF PUBLIC WORKS.	2

H.2.1	National Department of Public Works Documents	2

H.2.2	South African Buro of Standards (Standards and Specifications)	3

H.2.3	Acts, Regulations and Other Publications	5

H.3	THE CIDB AND THE CIVIL ENGINEER	7

H.3.1	Extracts From The Construction Industry Development Board Act, 2000, Act No.38 of 2000	7

H.3.2	Responsibilities of Consultant	8

H.3.3	CIDB Publications Relevant To The Civil Engineering Practice	9

H.4	OTHER RELEVANT SANS/SABS STANDARDS & SPECIFICATION	13

H.5	SITE LAYOUT PLAN:  DIRECTIVE TO THE CONSULTANT	43

H.5.1	Specifications	43

H.5.1.1	Title block	43

H.5.1.2	Drafting conventions	44

H.5.1.3	Services	46

H.5.1.4	Electronic format	47

H.5.1.5	Submission	48

H.6	FINAL ACCOUNT GUIDELINE	49

H.6.1	PRM044:  Finalisation of Final Account and Check List	54

H.6.2	PRM044:  Final Statement	55

H.6.3	PRM044:  Final Payment	56

# REFERENCE DOCUMENTS

GENERAL

In Sections A to E of this Manual reference is made to various other documents to be consulted in the use of this Manual for Civil Engineers 2011.  These documents are revised from time to time and the latest available version must be used.

On the tables below lists of reference documents and standard drawings are provided.  These tables will regularly be updated and the latest DPW documents published on the Departmental website:  http://www.publicworks.gov.za.

Updates of these tables are to be bound into this Manual.

REFERENCES TO DESIGN GUIDELINES AND OTHER INFORMATION DOCUMENTS TO BE USED FOR WORK FOR THE NATIONAL DEPARTMENT OF PUBLIC WORKS.

(Only the latest versions of the documents must be used)

National Department of Public Works Documents

| No. | NAME / DESCRIPTION | PUBLISHED BY | No. | DATE |
| --- | --- | --- | --- | --- |
| 1 | Appropriate Development of Infrastructure on Dolomite: Manual for Consultants | DPW | PW344 | Sep 2010 |
| 2 | Guidelines for the Design of Civil Services for Prisons | DPW | PW342 | Mar-04 |
| 3 | Standard Specification For Domestic And Fire Water Storage And Fire Water Supply For Public Buildings | DPW | PW345 | May-04 |
| 4 | Standard Technical Specification for a Pump Installation for Automatic Sprinkler Fire Extinguishing System | DPW | PW338 | Oct-81 |
| 5 | Civil Engineering Manual March 2011 | DPW | PW347/2011 | Mar 11 |
| 6 | Schematic Diagram for a Fire Pump Installation | DPW | PW340 | Dec 99 |
| 7 | Working Guideline for Project Managers ???? | DPW | Manual | Jun-03 |
| 8 | Preliminaries for inclusion in Bills of Quantities and Lump Sum Documents Based on the JBCC Series 2000 Documentation (Qs 003) | DPW | PW772/QS003 | Jun 2008 |
| 9 | Specification of Materials and Methods to be Used | DPW | PW371 | Oct-1993 |
| 10 | Small Waste Water Treatment Works | DPW | PW2011/1 |  |

South African Buro of Standards (Standards and Specifications)

| No. | NAME / DESCRIPTION | PUBLISHED BY | NO. | DATE |
| --- | --- | --- | --- | --- |
| 1 | Water supply and drainage for buildings Part 1: Water supply installations for buildings | SABS | SANS 10252-1:1993 (SABS 0252-1) | 1993 |
| 2 | Water supply and drainage for buildings Part 2: Drainage installations for buildings | SABS | SANS 10252-2:1993 | 1993 (2002) |
| 3 | Development, maintenance and management of groundwater resources Parts 1 to 9 | SABS | SANS 10299-1:2003 | 2003 |
| 4 | Community Protection against Fire: Outlines a system of determining the requirements for the operational and fire safety functions of emergency services rendered to communities. | SABS | SANS 10090:2003 (SABS 090) | 2003 |
| 5 | Code of Practice for the Wiring of Premises | SABS | SANS 10142-1 &2 :2009 | 2009 |
| 6 | The waterproofing of buildings (including damp-proofing and vapour barrier installation) | SABS | SANS 10021 (SABS 021) | 2006 |
| 7 | Application of National Building Regulations | SABS | SANS 10400 SABS 0400 | 1990 |
| 8 | Quality management systems - Requirements | SABS | SANS 9001 (SABS 9001 ISO 9001) | 2000 |
| 9 | Unplasticised polyvinyl chloride (PVC-U) sewer and drain pipes and pipe fittings. - | SABS | SANS 791:2004 (2009-07-31) | 2002 |
| 10 | The material used for water and fire mains must be specified to be at least class 9 uPVC according to SABS 966 (SANS 966) or class 10 HDPE to SABS ISO 4427 (SANS 4427). | SABS | SANS 966-1: 2010 SANS 4427:1996 / ISO 4427:1996 | 2010 & 2009 |
| 11 | Concrete non-pressure pipes. - | SABS | SANS 677:2003 | 2003 |
| 12 | Precast concrete manhole sections and components: | SABS | SANS 1294 | 2006 |
| 13 |  |  |  |  |
| 14 | The SANS 9000 family of standards on Quality Management Systems. | SABS | SANS 9000 | 2005 |
| 15 | SANS 9001, Quality management systems – Requirements. | SABS | SANS 9001 | 2000 |
| 16 | SANS 10120 (set): Code of practice for use with standardized specifications for civil engineering and contract document. | SABS | SANS 10120 | 1981 - 1996 |
| 17 | SANS 10400, The application of the National Building Regulations. | SABS | SANS 10400 | 1990 |
| 18 | SANS 1200 set (Standardized Specification for Civil Engineering Construction) and SANS 2001 set (Construction Works). | SABS | SANS 1200 | 1981 - 1990 |
| 19 | The SANS 14000 family of standards on Environmental Management Systems. | SABS | SANS 1400 | 2005 |
| 20 | SANS 14001, Environmental management systems – Requirements with guidance for use. | SABS | SANS 14001:2005 / ISO 14001 | 2005 |
| 21 | SANS 16001, HIV and AIDS management systems – General requirements. | SABS | SANS 16001 | 2007 |
| 22 | SANS 19011, Guidelines for quality and/or environmental management systems auditing. | SABS | SANS 19011 / ISO 19011 | 2003 / 2008 |

Acts, Regulations and Other Publications

| No | NAME / DESCRIPTION | PUBLISHED BY | NO. | DATE |
| --- | --- | --- | --- | --- |
| 1 | National Water Act 1998, and Regulations - | RSA Government | Act 36 of 1998 | 1998 |
| 2 | Water Services Act 1997, and Regulations - | RSA Government | Act 108 of 1997 | 1997 |
| 3 | Quality of Domestic Water Supplies, Vol. 1: Assessment Guide (1998). | Water Research Commission | Vol 1 | 1988 |
| 4 | Environment Conservation Act 1989, and Regulations | RSA Government | Act 73 of 1989 | 1989 |
| 5 | National Environmental Management Act 1998, and Regulations · (Act 107 of 1998); | RSA Government | Act 107 of 1998 | 1998 |
| 6 | National Environmental Management: Protected Areas Amendment Act (No. 15 of 2009) | RSA Government | Act 15 of 2009 | 2009 |
| 7 | National Environmental Management: Integrated Coastal Management Act (No. 24 of 2008) - 11 February 2009 No. 31884 | RSA Government | Act 24 of 2008 | 2009 |
| 8 | National Environmental Management: Waste act. 2008 - Government Gazette No. 32000, 10 March 2009 | RSA Government | - Act No. 59, 2008 | 2009 |
| 9 | Environmental Conservation Act, 1989 | RSA Government | Act 73 of 1989 | 1989 |
| 10 | Minimum Requirements for Waste Disposal by Landfill | Department of Water Affairs | Second Edition | 1998 |
| 11 | Minimum Requirements For Water Monitoring At Waste Management Facilities | Department of Water Affairs |  | 1998 |
| 12 | Minimum Requirements for the Handling, Classification and Disposal of Hazardous Waste | Department of Water Affairs |  | 1998 |
| 13 | Atmospheric Pollution Prevention Act, 1965 | RSA Government | Act 45 of 1965 | 1965 |
| 14 | Occupational Health and Safety Act, 1993 (Act 85 of 1993); | RSA Government | Act 85 of 1993 | 1993 |
| 15 | National Heritage Resources Act, 1999 ; GG No. 19974 28 APRIL 1999 | RSA Government | Act 25 of 1999 | 1999 |
| 16 | Post Office Act, 1998 (telephone installations); | RSA Government | Act 124 of 1998 | 1989 |
| 17 | Local. Government: Municipal Systems Act, 2000 | RSA Government | Act No. 32, 2000 | 2000 |
| 18 | Local Government Ordinance, 1939 (Ordinance No. 17 of 1939) | RSA Government | Ordinance 17 of 1939 | 1939 |
| 19 | Fire Brigade Services Act, 2000 | RSA Government | Act 14 of 2000 | 2000 |
| 20 | Electricity Act, 1996 | RSA Government | Act 88 of 1996 | 1996 |
| 21 | National Building Regulations and Building Standards Act 1996, and Regulations as amended by Standards Act 30 of 1982, NBR &BS Amendment Act 36 of 1984, NBR &BS Amendment Act 62 of 1989, NBR &BS Amendment Act 49 of 1995, | RSA Government | Act 103 of 1977 | 1995 |
| 22 | Minerals Act 1991, and Regulations | RSA Government | Act 50 of 1991 | 1991 |
| 24 | Guidelines for road construction materials | Department of Transport | TRH 14 | 1996 |
| 25 | Subsurface Drainage for Roads | Department of Transport | TRH 15 | 1995 |
| 26 | Council for the Built Environment Act. 2000, GG No. 2181X of 1 December 2000 | RSA Government | Act 43 of 2000 | 2000 |
| 27 | The Construction Industry Development Board Act 2000. (Act 38 of 2000) | RSA Government | Act 38 of 2000 | 2000 |
| 28 | Standard for Uniformity in Construction Procurement - GG No. 26427 of 9 JUNE 2004 | RSA Government | NOTICE 62 OF 2004 | 2004 |
| 29 | Skills Development Act No. 97 OF 1998 20 October, 1998 - Skills Development Amendment Act (No. 37 of 2008) - No. 37 of 2008: Skills Development Amendment Act, 2008. | RSA Government | 1 December 2008 No. 31666 | 2008 |
| 30 | General Conditions of Contract for use in connection with Works of Civil Engineering Construction | SAICE | GCC 2004 | 2004 |
| 31 | Guidelines for Human settlement Planning and Design | CSIR | Boutek Report No. BOU/E2001 | 2003 |
| 32 | Procurement Of Consulting Engineering Services In The Construction Industry: A Guide For Consulting Engineering Firms. | CESA – Consulting Engineers South Africa | PITT Manual | August 2009 |

THE CIDB AND THE CIVIL ENGINEER

Extracts From The Construction Industry Development Board Act, 2000, Act No.38 of 2000

“Definitions:

“construction industry” means the broad conglomeration of industries and sectors which add value in the creation and maintenance of fixed assets within the built environment;

“construction works” means the provision of a combination of goods and services arranged for the development, extension, installation, repair, maintenance, renewal. removal, renovation, alteration, dismantling or demolition of a fixed asset including building and engineering infrastructure;

“project” means a construction works contract or o series of related construction work contracts:

Objects of Board

4.	The objects of the Board are to—

- promote the contribution of the construction industry in meeting national construction demand and in advancing—

- national, social and economic development objectives;

- industry performance, efficiency and competitiveness; and

- improved value to clients;

- provide strategic leadership to construction industry stakeholders to stimulate sustainable growth, reform and improvement of the construction sector;

- determine and establish best practice that promotes—

- improved industry stability;

- improved industry performance, efficiency and effectiveness;

- procurement and delivery management reform;

(iv)	improved public sector delivery management;

- national social and economic objectives, including—

(aa)	growth of the emerging sector;

(bb)	labour absorption in the construction industry;

(cc)	improved Iabour relations; and

(dd)	positive safety, health and environmental outcomes;

- human resource development in the construction industry;

- promote best practice through the development and implementation of appropriate programmed and measures aimed at best practice and improved performance of public and private sector clients, contractors and other participants in the construction delivery process;

- promote uniform application of policy with regard to the construction industry throughout all spheres of Government;

- promote, establish or endorse—

- uniform standards; and

- ethical standards,

that regulate the actions, practices and procedures of parties engaged in construction contracts;

Powers. functions and duties of Board

5. (1)	To provide strategic leadership, the Board—

(a)	must promote and implement policies. programmed and projects aimed at, amongst others—

(i)	support of emerging enterprises sector; 5

(ii)	work process transformation;

- improved Iabour relations:

(iv)	human resource development;

- innovation and best practice processes;

- simplification of regulatory procedures;

- procurement reform;

- standardisation and uniformity in procurement documentation, practices and procedures;

(ix)	best practice and value for money with regard to design;

(x)	regional co-operation; and 15

(xi)	international competitiveness:

(b)	may initiate policy refinement and development in cooperation with Government:

(c)	must facilitate communication between construction industry stakeholders, all spheres of Government and statutory bodies:

(d)	must establish a construction industry stakeholders” forum as contemplated in 	section 13; and

(e)	must provide information to stakeholders on best practice, industry performance and improvement and generally on matters affecting the construction industry.

(2)	To promote best practice, the Board- may establish and maintain a register of 	suppliers, manufacturers or service providers in the construction industry. The 	provisions of Chapter Four apply with the necessary changes required by the 	context, to that register;

(3)	To advance [he uniform application of policy with regard to construction industry development, the Board-

(c)	must. within the framework of the procurement policy of Government. promote the standardisation of the procurement process with regard to the construction industry:

(4)	To promote uniform and ethical standards within the construction industry, the Board-

(a)	must publish a code of conduct for all construction-related procurement and all 	participants involved in the procurement process; and

(b)	may initiate, promote and implement national programmed and projects aimed at the 	standardisation of procurement documentation, practices and procedures.

Responsibilities of Consultant

- Advise CIDB of all tenders advertised (Chapter 4)

- Advise CIDB of any contractors contravening their registration status.

- Compiling tenders:

“6.1 General

The CIDB Standard for Uniformity in Construction Procurement requires that employers, wherever  appropriate, in support of industry development, shall in the application of the register of contractors, promote the participation and development of registered contractors by means of one or more of the following:

a)	unbundling projects into smaller contracts;

b)	implementing targeted development programmes to support potentially emerging contractors in accordance with the provisions of the Construction Industry Development Regulations;

c)	offering preferences to contractors who undertake to enter into joint ventures with or subcontract portions of the works to such contractors; and

d)	requiring a prime contractor to subcontract defined portions of the works to such contractors in accordance with the provisions of the Standard set out in Annex I.

In addition, employers can offer preferences in terms of a preferential procurement policy to a contractor registered in particular contractor grading designations.

Accordingly, there are a number of contracting and targeting strategies that employers can use to promote the development of registered contractors1. These strategies enable registered contractors to qualify for registration in a higher contracting grading designation by providing them with work opportunities so that they can improve upon their track record.”

Refer in this regard also to the CESA Guideline for the 'Procurement of Consulting Engineering Services'

CIDB Publications Relevant To The Civil Engineering Practice

CIDB – Practice Manual – “Implementing employment intensive road works” – available on the CIDB website (http://www.cidb.org.za)

CIDB – Best Practice Guidelines – “Labour Based Methods and Technologies for Employment Intensive Construction Works” obtainable from the South African Institution of Civil Engineers

CIDB Practice Note 22b: CIDB Infrastructure Gateway System

Requirements of Local Authority where the relevant authority provides the fire prevention and fire fighting services.  In the case where the Local Authority’s requirements are more stringent than those described in this document, the Local Authority requirements shall prevail.

Standard for Uniformity in Construction Procurement and Best Practice Guidelines

May 2010 This standard establishes requirements for procurement within the construction industry which are aimed at bringing about standardisation and uniformity in construction

procurement documentation, practices and procedures.

Code of conduct for all parties engaged in Construction Procurement
October 2003

Good corporate governance is being introduced throughout the world as a business imperative offering competitive advantage. Investors and employers prefer to do business with those business enterprises that uphold the key elements of corporate governance, namely discipline, transparency, independence, accountability, responsibility, fairness and social responsibility. The CIDB in consultation with Industry Stakeholders has developed a Code of Conduct for all parties engaged in Construction Procurement.

Practice Note 21: Bills of Quantities (Version 1 - August 2010)

Bills of Quantities (BoQ) are the most common form of pricing strategy used where the contractor undertakes construction works on the basis of full designs issued by the employer. This practice note outlines what a bill of quantities is and provides a recommended procedure for evaluating tenders where the prices are based on a BoQ. The recommended procedure suggests that a priced BoQ is not included in the tender submission and is only called for from the highest ranked or scoring tenderer during the tender evaluation process so that the acceptability of the rates can be confirmed prior to the award of a contract.

Practice Note 20: Applying the Register of Contractors to Fencing (Version 1 - 
July 2010)

Fences and walls are required to enclose, protect or to divide a space or property. Such construction works can fall into a number of classes of construction works. This practice notes provides practical guidance on the identification of an appropriate class of construction works to apply the register of contractors to such works.

Practice Note 15: Framework Agreements (Version 2 – October 2008)

This practice note outlines the nature and features of framework agreements, describes the essential elements of and the selection procedures associated with and identifies the forms of contract that are best suited to such contracts.

Code of conduct for all parties engaged in Construction Procurement
October 2003

Good corporate governance is being introduced throughout the world as a business imperative offering competitive advantage. Investors and employers prefer to do business with those business enterprises that uphold the key elements of corporate governance, namely discipline, transparency, independence, accountability, responsibility, fairness and social responsibility. The CIDB in consultation with Industry Stakeholders has developed a Code of Conduct for all parties engaged in Construction Procurement.

Practice Note 21: Bills of Quantities (Version 1 - August 2010)

Bills of Quantities (BoQ) are the most common form of pricing strategy used where the contractor undertakes construction works on the basis of full designs issued by the employer. This practice note outlines what a bill of quantities is and provides a recommended procedure for evaluating tenders where the prices are based on a BoQ. The recommended procedure suggests that a priced BoQ is not included in the tender submission and is only called for from the highest ranked or scoring tenderer during the tender evaluation process so that the acceptability of the rates can be confirmed prior to the award of a contract.

Practice Note 11: Programme Management Approach to Delivery (Version 1 – February 2008)

This Practice Note discusses a delivery model using a programme management approach that enables allocated budgets to be spent in a manner that results in construction works of an acceptable quality in response to prioritised needs, whilst contributing to the regional social and economic agenda including employment and skills development.

Practice Note 10: Attaining Social and Economic Deliverables (Version 1 - February 2008)
This Practice Note discusses a number of targeted procurement procedures relating to the measurement and quantification of the participation of target groups, the provision of incentives for attaining key performance indicators in the performance of a contract and the creation of contractual obligations to engage target groups in the performance of the contract. It also outlines the approach adopted in the CIDB Specification for Social and Economic Deliverables in Construction Works Contracts for the delivery of a wide range of social and economic deliverables including the employment of local resources, employment opportunities for labour intensive works, business opportunities, enterprise support and development programmes and skills development.

Practice Note 7: Subcontracting Arrangements (Version 1 - May 2007)

This Practice Note examines the different types of subcontractors that are encountered in construction works, namely nominated, selected and domestic. It presents an overview of the subcontracting provision of the FIDIC, GCC 2004, JBCC Series 2000 and NEC3 standard forms of contract as well as the CIDB registration requirements and other public sector requirements.

Practice Note 2: Correcting Arithmetical Errors in Tenders (Version 3 – February 2008)

This Practice Note sets out the manner in which arithmetical corrections are to be made in the evaluation of tenders in accordance with the CIDB Standard Conditions of Tender.

Practice Note 1: Scaling up Delivery and Accelerating Empowerment (version 1 - August 2006)

This Practice Note identifies systemic problems in scaling up delivery and accelerating empowerment, and outlines fresh approaches in contracting strategies that increase the size and duration of contracts to enable: 1. enhanced delivery and the efficient use of resources; 2. sustainable enterprise growth, employment, workforce skills development and sustainable empowerment.

Implementing Employment Intensive Road Works - A CIDB Practice Manual
March 2007

The aim of this manual is to provide contractors, involved in the labour-based construction and upgrading of low volume roads, with an introduction to and basic understanding of mathematical concepts and calculations; survey concepts and calculations; materials and materials related concepts and typical road terms and components encountered in the construction of these roads.

Labour-based Methods and Technologies for Employment Intensive Construction Works - A CIDB Guide to Best Practice
March 2005

These best practice guidelines present current state-of-the-art practices in a wide range of labour-based construction methods, manufacturing methods and technologies which have been successfully utilised in South Africa in recent years. They are intended to provide sufficient technical information on such methods and technologies to enable those responsible for the design of projects to make confident and informed choices on their use in project.

Toolkit for Infrastructure Delivery Management

The Infrastructure Delivery Management Toolkit has been developed through collaboration between National Treasury and the CIDB as the basis for consistent and improved public sector infrastructure delivery. It provides a systematic approach to infrastructure delivery covering the full cycle from needs identification, planning and budgeting through to procurement, construction, handover and maintenance. It also provides appropriate procedures and guidelines for delivery managers in the public sector by linking them to relevant policy, legislation and regulation that underpin the planned implementation of government’s infrastructure.

Construction Health and Safety in South Africa; Status and Recommendations
June 2009

Construction health and safety has long been the focus of attention of many industry stakeholders and role-players in South Africa, and while it is acknowledged that many industry associations and professional societies, contracting organisations and others have made significant efforts to improve health and safety within the construction industry, overall, construction health and safety is not improving significantly. Notably, construction continues to contribute a disproportionate number of fatalities and injuries, and there continues to be a high level of non-compliance with the health and safety Regulations in South Africa. Against this context, the CIDB has undertaken this report on the status of construction health and safety in South Africa, so as to provide a context for the efforts and actions of industry stakeholders and role-players in improving construction health and safety – including those of the CIDB.

The National Infrastructure Maintenance Strategy in Support of ASGISA and Government Growth Objectives
July 2007

The National Infrastructure Maintenance Strategy (NIMS) is a co-ordinated programme of actions that is an essential part of governments vision of delivering infrastructure services to all. It was approved by Cabinet on 23 August 2006, and provides the proposed institutional arrangements for facilitating and managing the NIMS programme, key work components and task teams to address the actions set down within NIMS and a summary timetable for mobilizing the programme.

OTHER RELEVANT SANS/SABS STANDARDS & SPECIFICATION

| No. | OTHER RELEVANT SANS/SABS STANDARDS & SPECIFICATIONS |  |  |  |
| --- | --- | --- | --- | --- |
|  | Aggregates of low density - Covers inorganic aggregates of low density for use in structural and insulating concretes and in plaster. | SABS | SANS 794:2009 |  |
|  | Bituminous damp-proof courses - Covers the following three (3) types of damp-proof course for use in the waterproofing of buildings: Type AA: Damp-proof course with a base of asbestos-felt and a net mass of at least 33,5 kg/9 square metre. Type FV: Damp-proof course with a base fibre-felt and a net mass of at least 57,0 kg/18 square metre. Type GH: Damp-proof course with a base of hessian and net mass of at least 33,5 kg/9 square metre. | SABS | SANS 248:2007 |  |
|  | Penetration grade bitumens - Covers four (4) penetration grades of bitumen suitable for road construction and similar purposes. | SABS | SANS 307:2005 (2010-08-20) |  |
|  | Anionic bitumen road emulsions - Covers physical and chemical properties of anionic bitumen road emulsions, for applications on roads and other traffic surfaces. | SABS | SANS 309:2004 |  |
|  | Industrial bitumens - Covers the ten (10) most widely used grades of bitumen (3 grades of the penetration grade type and 7 grades of the blown type) for industrial purposes. | SABS | SANS 317:2007 |  |
|  | Cationic bitumen road emulsions - Covers the requirements for three types (of the appropriate grade(s) given in table 1) of cationic bitumen road emulsions suitable for the preparation and treatment of road and other surfaces carrying wheeled and foot traffic. | SABS | SANS 548:2003 |  |
|  | Bituminous aluminium paint - Covers one type of bituminous aluminium paint for interior and exterior use on primed metal, masonry, asbestos cement and wood surfaces. It may also be applied to roofing felt complying with SABS 92, to creosoted timber, and to hard bituminous surfaces that have been allowed to weather for at least eight (8) weeks before painting. | SABS | SANS 802:2001 (2006-04-19) |  |
|  | Metal ties for cavity walls - Covers the dimensional and material requirements of metal ties suitable for walls having cavity widths of up to the largest likely to be found in masonry walling constructed to dimensionally co-ordinated sizes. | SABS | SANS 28:2010 |  |
|  | Burnt clay masonry units - Covers burnt clay masonry units for use in masonry walling (engineering, facing and non-facing). Lays down a classification system. Includes requirements for shape, appearance, texture, colour, dimensions, warpage, compressive strength, efflorescence and soundness; methods of test for water absorption, water-soluble salts, and moisture expansion and information on compressive strength of engineering units, the usage rate of units per square metre of walling and explanatory notes on efflorescence. | SABS | SANS 227:2007 |  |
|  | Calcium silicate masonry units - Specifies the characteristics of masonry units of the calcium silicate type for use in masonry walling. | SABS | SANS 285:2010 |  |
|  | Limes for use in building - Covers quicklime and slaked (hydrated) limes for use in building and produced by calcining of limestone, or treatment of calcium carbide. | SABS | SANS 523:2009 |  |
|  | Methods of testing cement Part 1: Determination of strength - Describes the method for the determination of the compressive strength and, optionally, the flexural strength of cement mortar. | SABS | SANS 50196-1:2006 / EN 196-1:2005 |  |
|  | Methods of testing cement Part 2: Chemical analysis of cement - Specifies the methods for the chemical analysis of cement. It also describes the reference methods and, in certain cases, an alternative method which can be considered to be equivalent. In the case of a dispute, however, only the reference methods are used. | SABS | SANS 50196-2:2006 / EN 196-2:2005 |  |
|  | Methods of testing cement Part 3: Determination of setting times and soundness - Specifies the methods for determining standard consistence, setting times and soundness of cement. It applies to common cements and to other cements and materials. It may not apply to other cement types that have, for example, a very short initial setting time. It is used for assessing whether the setting time and soundness of a cement are in conformity with its specification. | SABS | SANS 50196-3:2006 / EN 196-3:2005 |  |
|  | Methods of testing cement Part 4: Quantitative determination of constituents - Lays down the procedures for determining the contents of the constituents of cements that fall within the scope of ENV 197-1. | SABS | SANS 50196-4:1993 (2007-03-02) |  |
|  | Methods of testing cement Part 5: Pozzolanicity test for pozzolanic cement - Specifies the method of measuring the pozzolanicity of pozzolanic cements conforming to EN 197-1. Does not aply to Portland pozzolana cements or to pozzolanas. | SABS | SANS 50196-5:2009 |  |
|  | Methods of testing cement Part 6: Determination of fineness - Describes two methods of determining the fineness of cement. Applicable to all the cements defined in ENV 197. | SABS | SANS 50196-6:1989 (2009-01-23) |  |
|  | Methods of testing cement Part 7: Methods of taking and preparing samples of cement - Describes the equipment to be used, the methods to be followed and the provisions for taking samples of cement, representative of given lots of testing, to assess the quality of products prior to, during or after delivery. | SABS | SANS 50196-7:2009 / EN 196-7:2007 |  |
|  | Cement Part 1: Composition, specifications and conformity criteria for common cements - Defines and gives the specifications of 27 distinct common cement products and their constituents. The definition of each cement includes the proportions in which the constituents are to be combined to produce these distinct products in a range of six strength classes. The definition also includes requirements the constituents have to meet and the mechanical, physical and chemical requirements including, where appropriate, heat of hydration requirements of the 27 products and strength classes. It also states the conformity criteria and the related rules. Necessary durability requirements are also given. | SABS | SANS 50197-1:2000 / EN 197-1:2000 (2006-04-07) |  |
|  | Cement Part 2: Conformity evaluation - Specifies the scheme for the evaluation of conformity of cements to their corresponding product specification standard, including certification of conformity by a certification body. | SABS | SANS 50197-2:2000 / EN 197-2:2000 (2006-09-08) |  |
|  | Masonry cement Part 1: Composition, specifications and conformity criteria - Gives the definition and covers the composition of masonry cements used for the production of mortar for bricklaying, and for plaster. Gives physical, mechanical, chemical and durability requirements and defines strength classes. Also states the conformity criteria and the related rules. | SABS | SANS 50413-1:2004 / EN 413-1:2004 (2005-03-18) |  |
|  | Masonry cement Part 2: Test methods - Describes reference and alternative test methods to be used when testing masonry cements to assess their conformity to SANS 50413-1. It gives the tests on fresh mortar for consistence, water retention and air content. | SABS | SANS 50413-2:2006 / EN 413-2:2005 |  |
|  | Flat products made of steels for pressure purposes Part 1: General requirements - Specifies the general technical delivery conditions for flat products used principally for the construction of pressure equipment. | SABS | SANS 50028-1:2004 / EN 10028-1:2000 (2009-05-29) |  |
|  | Flat products made of steels for pressure purposes Part 2: Non-alloy and alloy steels with specified elevated temperature properties - Specifies requirements for flat products for pressure equipment made of weldable non-alloy and alloy steels with elevated temperature properties as specified in table 1. | SABS | SANS 50028-2:2009 / EN 10028-2:2009 |  |
|  | Flat products made of steels for pressure purposes Part 3: Weldable fine grain steels, normalized - Specifies requirements for flat products for pressure equipment made of weldable fine grain steels as specified in table 1. | SABS | SANS 50028-3:2009 / EN 10028-3:2009 |  |
|  | Flat products made of steels for pressure purposes Part 4: Nickel alloy steels with specified low temperature properties - Specifies requirements for flat products for pressure equipment made of nickel alloy steels as specified in table 1. | SABS | SANS 50028-4:2009 / EN 10028-4:2009 |  |
|  | Flat products made of steels for pressure purposes Part 5: Weldable fine grain steels, thermomechanically rolled. - Specifies requirements for flat products for pressure equipment made of thermomechanically rolled steels as specified in table 1. | SABS | SANS 50028-5:2009 / EN 10028-5:2009 |  |
|  | Flat products made of steels for pressure purposes Part 6: Weldable fine grain steels, quenched and tempered. - Specifies the requirements for flat products for pressure equipment made of quenched and tempered steels as specified in table 1. | SABS | SANS 50028-6:2009 / EN 10028-6:2009 |  |
|  | Flat products made of steels for pressure purposes Part 7: Stainless steels - Specifies requirements for flat products for pressure purposes made of stainless steels, including austenitic creep resisting steels. | SABS | SANS 50028-7:2005 |  |
|  | Products and systems for the protection and repair of concrete structures - Test methods - Determination of creep in compression for repair products - Specifies a method for measuring creep under compressive load in products and systems for the repair of concrete structures, as defined in prEN 1504-3. Suitable for repair mortars and concretes with polymer binders (PC) and repair grouts, mortars and concretes with hydraulic binders (CC) and polymer cement binders (PCC). | SABS | SANS 53584:2010 / EN 13584:2003 |  |
|  | Products and systems for the protection and repair of concrete structures - Test methods - Determination of creep in compression for repair products - Specifies a method for measuring creep under compressive load in products and systems for the repair of concrete structures, as defined in prEN 1504-3. Suitable for repair mortars and concretes with polymer binders (PC) and repair grouts, mortars and concretes with hydraulic binders (CC) and polymer cement binders (PCC). | SABS | SANS 53584:2010 / EN 13584:2003 |  |
|  | Cranes - Competency requirements for crane inspectors - Specifies the competency required for persons who carry out periodic, exceptional, alteration and thorough inspections of cranes. It excludes the inspection and checks performed by crane operators and maintenance personnel. | SABS | SANS 23814:2009 / ISO 23814:2009 |  |
|  | Polyolefin film for damp- and waterproofing in buildings - Covers the requirement for 6 types of polyolefin film for use as a damp-proofing material in walls, under concrete and roofing tiles, and or the waterproofing of basements. Specifies requirements for dimensions; tensile properties; resistance to puncture, aging and weathering; water penetration and light transmittance. An appendix contains information on the use and protection of the film. | SABS | SANS 952:1985 (2000-11-15) |  |
|  | Earth-moving machinery - Basic types - Identification and terms and definitions - Gives terms and definitions and an identification structure for classifying earth-moving machinery designed to perform the following operations: excavation, loading, transportation, and drilling, spreading, compacting or trenching of earth and other materials, for example, during work on roads and dams, and on building sites. Provides a clear means of identifying machines according to their function and design configurations. | SABS | SANS 261:2008 / ISO 6165:2006 |  |
|  | Earth-moving machinery - Safety Part 13: Requirements for rollers - Specifies the safety requirements specific to rollers, and the appropriate technical measures for eliminating or reducing risks arising from significant hazards, hazardous situations or events during commissioning, operation and maintenance. | SABS | SANS 20474-13:2010 / ISO 20474-13:2008 |  |
|  | Earth-moving machinery - Safety Part 2: Requirements for tractor-dozers - Covers the safety requirements specific to wheeled and crawler tractor-dozers, as well as those for rear-mounted winches on dozers. | SABS | SANS 20474-2:2010 / ISO 20474-2:2008 |  |
|  | Earth-moving machinery - Safety Part 3: Requirements for loaders - Specifies the safety requirements specific to loaders, and the appropriate technical measures for eliminating or reducing risks arising from significant hazards, hazardous situations or events during commissioning, operation and maintenance. | SABS | SANS 20474-3:2010 / ISO 20474-3:2008 |  |
|  | Earth-moving machinery - Safety Part 4: Requirements for backhoe loaders - Specifies the safety requirements specific to wheeled and crawler backhoe loaders, and deals with all significant hazards, hazardous situations and events relevant to the earth-moving machinery within its scope when used as intended or under conditions of misuse reasonably foreseeable by the manufacturer | SABS | SANS 20474-4:2010 / ISO 20474-4:2008 |  |
|  | Earth-moving machinery - Safety Part 5: Requirements for hydraulic excavators - Specifies the safety requirements specific to hydraulic excavators, and the appropriate technical measures for eliminating or reducing risks arising from significant hazards, hazardous situations or events during commissioning, operation and maintenance. | SABS | SANS 20474-5:2010 / ISO 20474-5:2008 |  |
|  | Earth-moving machinery - Safety Part 6: Requirements for dumpers - Specifies the safety requirements specific to wheeled and crawler dumpers, including compact dumpers and compact dumpers with standing operator. It is not applicable to road-truck-mounted dumpers. | SABS | SANS 20474-6:2010 / ISO 20474-6:2008 |  |
|  | Earth-moving machinery - Safety Part 7: Requirements for scrapers - Specifies the safety requirements specific to wheeled and crawler scrapers, except for towed scrapers, and the appropriate technical measures for eliminating or reducing risks arising from significant hazards, hazardous situations or events during commissioning, operation and maintenance. | SABS | SANS 20474-7:2010 / ISO 20474-7:2008 |  |
|  | Earth-moving machinery - Safety Part 8: Requirements for graders - Specifies the safety requirements specific to graders, and deals with all significant hazards, hazardous situations and events relevant to the earthmoving machinery within its scope when used as intended or under conditions of misuse reasonably foreseeable by the manufacturer. | SABS | SANS 20474-8:2010 / ISO 20474-8:2008 |  |
|  | Earth-moving machinery - Safety Part 9: Requirements for pipelayers - Specifies the safety requirements specific to pipelayers, and the appropriate technical measures for eliminating or reducing risks arising from significant hazards, hazardous situations or events during commissioning, operation and maintenance. | SABS | SANS 20474-9:2010 / ISO 20474-9:2008 |  |
|  | Explosives for civil uses - High explosives Part 14: Determination of velocity of detonation - Specifies a method for determining the velocity of detonation of an explosive. The method is applicable to explosives for civil uses in cartridged or bulk form. | SABS | SANS 53631-14:2006 / EN 13631-14:2003 |  |
|  | Explosives for civil uses - High explosives Part 14: Determination of velocity of detonation - Specifies a method for determining the velocity of detonation of an explosive. The method is applicable to explosives for civil uses in cartridged or bulk form. | SABS | SANS 53631-14:2006 / EN 13631-14:2003 |  |
|  | Explosives for civil uses - Detonators and relays Part 1: Requirements - Explosives for civil uses - This standard specifies the requirements for detonators, surface connectors, shock tube and detonating cord relays for civil uses. It does not cover electronic initiation systems (see SANS 53763-27) nor the devices and accessories for reliable and safe function of detonators and relays (see SANS 53763-26). | SABS | SANS 53763-1:2008 / EN 13763-1:2004 |  |
|  | Explosives for civil uses - Detonators and relays Part 11: Determination of resistance to damage by dropping of detonators and relays - Specifies methods for checking that a detonator, relay or surface connector will not explode when accidentally dropped onto a hard surface. | SABS | SANS 53763-11:2008 / EN 13763-11:2003 |  |
|  | Explosives for civil uses - Detonators and relays Part 12: Determination of resistance to hydrostatic pressure - Defines the methods for the determination of resistance to hydrostatic pressure of electric and non-electric detonators, surface connectors and relays. | SABS | SANS 53763-12:2006 / EN 13763-12:2003 |  |
|  | Explosives for civil uses - Detonators and relays Part 13: Determination of resistance of electric detonators to electrostatic discharge - Specifies a method for determining whether electric detonators can withstand an electrostatic discharge (ESD) without detonating. The method is not applicable to magnetically-coupled detonators. | SABS | SANS 53763-13:2006 / EN 13763-13:2004 |  |
|  | Explosives for civil uses - Detonators and relays Part 15: Determination of equivalent initiating capability - Specifies a method of determining the equivalent initiating capability of detonators. It also specifies a functioning test (after storage) at high and low temperatures. The document is not applicable to surface connectors or detonating cord relays. | SABS | SANS 53763-15:2008 / EN 13763-15:2004 |  |
|  | Explosives for civil uses - Detonators and relays Part 16: Determination of delay accuracy - Describes a method for determining the delay time accuracy of detonators and surface connectors with pyrotechnic delay elements. It does not apply to those with an electronic time delay. | SABS | SANS 53763-16:2006 / EN 13763-16:2003 |  |
|  | Explosives for civil uses - Detonators and relays Part 17: Determination of no-fire current of electric detonators - Describes a method for determining the no-fire current of electric detonators. | SABS | SANS 53763-17:2006 / EN 13763-17:2003 |  |
|  | Explosives for civil uses - Detonators and relays Part 18: Determination of series firing current of electric detonators - Describes a method to determine the reliability of electric detonators connected in series, when the manufacturer's stated series firing current is applied. The test method does not apply to magnetic coupled detonators and electronic detonators. | SABS | SANS 53763-18:2006 / EN 13763-18:2003 |  |
|  | Explosives for civil uses - Detonators and relays Part 19: Determination of firing impulse of electric detonators - Specifies a method for determining the all-fire-impulse and the no-fire impulse of electric detonators | SABS | SANS 53763-19:2008 / EN 13763-19:2003 |  |
|  | Explosives for civil uses - Detonators and relays Part 2: Determination of thermal stability - Specifies a method for determining the thermal stability of electric detonators, non-electric detonators, surface connectors, detonating cord relays and shock tubes for use with non-electric detonators. | SABS | SANS 53763-2:2008 / EN 13763-2:2002 |  |
|  | Explosives for civil uses - Detonators and relays Part 20: Determination of total electrical resistance of electric detonators - Defines a method for determining the electrical resistance of electric detonators. | SABS | SANS 53763-20:2006 / EN 13763-20:2003 |  |
|  | Explosives for civil uses - Detonators and relays Part 21: Determination of flash-over voltage of electric detonators - Specifies a method of determining the flash-over voltage of electric detonators. | SABS | SANS 53763-21:2008 / EN 13763-21:2003 |  |
|  | Explosives for civil uses - Detonators and relays Part 24: Determination of the electrical non-conductivity of shock tube - Specifies methods of determining the electrical insulation resistance (non-conductivity) and the electrical flash-over distance of shock tubes for use with non-electric detonators. | SABS | SANS 53763-24:2008 / EN 13763-24:2002 |  |
|  | Explosives for civil uses - Detonators and relays Part 26: Definitions, methods and requirements for devices and accessories for reliable and safe function of detonators and relays - This standard specifies the constructional and functional requirements for testing of devices and accessories needed for the reliable and safe initiation of detonators and relays, under normal working conditions. | SABS | SANS 53763-26:2008 / EN 13763-26:2004 |  |
|  | Explosives for civil uses - Detonators and relays Part 27: Definitions, methods and requirements for electronic initiation systems - This standard specifies a risk analysis, evaluation and testing procedure to be used to investigate the safety and reliability of electronic initiation systems by identifying hazards and estimating the risks associated with the system. It also stipulates levels of acceptability for electronic initiation systems. | SABS | SANS 53763-27:2008 / CEN/TS 13763-27:2003 |  |
|  | Detonators and relays Part 3: Determination of sensitiveness to impact - Specifies methods for checking that plain detonators, electric detonators, non-electric detonators, surface connectors, shock tubes and detonating cord relays will not explode when subjected to an impact under specified conditions. | SABS | SANS 53763-3:2008 / EN 13763-3:2002 |  |
|  | Explosives for civil uses - Detonators and relays Part 4: Determination of resistance to abrasion of leading wires and shock tubes - Specifies a method for determining the resistance to abrasion of plastics used as insulating material for leading wires of electric detonators, or used as base material for tubing of shock tube non-electric detonators. | SABS | SANS 53763-4:2008 / EN 13763-4:2003 |  |
|  | Explosives for civil uses - Detonators and relays Part 5: Determination of resistance to cutting damage of leading wires and shock tubes - Specifies a method for determining the resistance to failure of the plastic materials used in insulation of leading wires of electric detonators, or the tubing of shock tube non-electric detonators, when drawn over a sharp edge under specified loading conditions. | SABS | SANS 53763-5:2008 / EN 13763-5:2003 |  |
|  | Explosives for civil uses - Detonators and relays Part 6: Determination of resistance to cracking in low temperatures of leading wires - Specifies a method for determining the resistance to cracking at low temperatures of leading wire insulation of detonators. It is only applicable to electric detonators with plastic covered leading wires. | SABS | SANS 53763-6:2008 / EN 13763-6:2003 |  |
|  | Explosives for civil uses - Detonators and relays Part 7: Determination of the mechanical strength of leading wires, shock tubes, connections, crimps and closures - Specifies a method for determining the ability of detonator leading wires/shock tubes, and their connections into the crimp/closure or sealing arrangement, to withstand a pullout when subjected to a pulling force. | SABS | SANS 53763-7:2008 / EN 13763-7:2003 |  |
|  | Explosives for civil uses - Detonators and relays Part 8: Determination of the resistance to vibration of plain detonators - Specifies a method for assessing the ability of the explosive content of plain detonators to withstand the vibration likely to be experienced in normal use and handling. | SABS | SANS 53763-8:2008 / EN 13763-8:2003 |  |
|  | Explosives for civil uses Part 1: Terminology - Defines the key technical terms used in the European Standards developed in the field of explosives for civil uses. | SABS | SANS 53857-1:2008 / EN 13857-1:2003 |  |
|  | Zinc-coated fencing wire - Specifies the characteristics of drawn steel wire, zinc-coated by the hot-dip process, to be used for line fencing wire or barbed fencing wire for general purposes. | SABS | SANS 675:2009 |  |
|  | Fences Part 12: Specification for steel palisade fences - Specifies requirements for steel palisade fences and gates of corrugated and angle pales for heights up to 2,4 m for general purposes and of corrugated pales for heights of 3,0 m and 3,6 m, all with posts at 2,75 m centers for security purposes. | SABS | SANS 301-12:2004 / BS 1722-12:1999 (2009-05-29) |  |
|  | E glass fibre chopped strand mat for reinforcement of polyester and other liquid laminating systems - Specifies requirements for E glass fibre chopped strand mat, the strands being laid at random and held together with a binder, for the reinforcement of liquid resin systems. | SABS | SANS 419:2007 |  |
|  | The design and construction of reinforced soils and fills - Gives recommendations for the application of reinforcement techniques to soils and other fills. Also gives design guidelines for the reinforcement of soils. | SABS | SANS 207:2006 |  |
|  | Concrete retaining blocks - Covers concrete blocks which are dry stacked into a wall which is used to retain earth, sand or stone and other superimposed loads. | SABS | SANS 508:2008 |  |
|  | Flexible polyurethane (polyester) foams - Covers the requirements for five classes (based on density) of flexible polyurethane foams of the polyester type, in the form of blocks, slabs, sheets, and shapes cut from these. Each class is subdivided, according to the hardness of the foam, into two grades. | SABS | SANS 641:2010 |  |
|  | Ergonomics data and guidelines for the application of ISO/IEC Guide 71 to products and services to address the needs of older persons and persons with disabilities - Presents ergonomics data and guidelines for applying ISO/IEC Guide 71 (published in South Africa as an identical adoption under the designation ARP 071) in addressing the needs of older persons and persons with disabilities in standards development. It provides ergonomics data and knowledge about human abilities (sensory, physical, cognitive abilities) and allergies, and guidance on the accessible design of products, services and environments. | SABS | SANS 22411:2009 / ISO/TR 22411:2008 |  |
|  | Elastomeric structural glazing and panel gaskets - Covers elastomeric structural glazing and panel gaskets systems used in building applications. Requirements for material, workmanship, dimensions and physical properties are specified. | SABS | SANS 635:2007 |  |
|  | Glass in building - Basic soda lime silicate glass products Part 1: Definitions and general physical and mechanical properties - Defines and classifies basic glass products, indicates their chemical composition, their main physical and mechanical characteristics and defines their general quality criteria. | SABS | SANS 50572-1:2006 / EN 572-1:2004 |  |
|  | Glass in building - Basic soda lime silicate glass products Part 2: Float glass - Specifies dimensional and minimum quality requirements (in respect of optical and visual faults) for float glass, as defined in SANS 50572-1, for use in building. Applies only to float glass supplied in jumbo sizes and split sizes. | SABS | SANS 50572-2:2006 / EN 572-2:2004 |  |
|  | Glass in building - Basic soda lime silicate glass products Part 3: Polished wire glass - Specifies dimensional and minimum quality requirements (in respect of optical, visual and wire faults) for polished wired glass, as defined in SANS 50572-1, for use in building. Applies only to polished wire glass supplied in rectangular panes and in stock sizes. | SABS | SANS 50572-3:2006 / EN 572-3:2004 |  |
|  | Glass in building - Basic soda lime silicate glass products Part 4: Drawn sheet glass - Specifies dimensional and minimum quality requirements (in respect of optical and visual faults) for drawn sheet glass, as defined in SANS 50572- 1, for use in building. Applies only to drawn sheet glass supplied in rectangular panes and in stock sizes. | SABS | SANS 50572-4:2006 / EN 572-4:2004 |  |
|  | Glass in building - Basic soda lime silicate glass products Part 5: Patterned glass - Specifies dimensional and minimum quality requirements (in respect of visual and pattern faults) for patterned glass, as defined in SANS 50572-1, for use in building. It applies only to patterned glass supplied in rectangular panes and in stock sizes. | SABS | SANS 50572-5:2006 / EN 572-5:2004 |  |
|  | Protection against lightning Part 1: General principles - Provides the general principles to be followed in the protection against lightning of, structures including their installations and contents as well as persons, and services connected to the structure | SABS | SANS 62305-1:2007 / IEC 62305-1:2006 |  |
|  | Protection against lightning Part 2: Risk management - Applies to risk assessment for a structure or a service due to lightning flashes to earth. Its purpose is to provide a procedure for the evaluation of such a risk. Once an upper tolerable limit for the risk has been selected, this procedure allows the selection of appropriate protection measures to be adopted to reduce the risk to or below the tolerable limit | SABS | SANS 62305-2:2007 / IEC 62305-2:2006 |  |
|  | Protection against lightning Part 3: Physical damage to structures and life hazard - Provides the requirements for the protection of a structure against physical damage by means of a lightning protection systems (LPS), and the protection against injury to living beings due to touch and step voltages in the vicinity of an LPS. | SABS | SANS 62305-3:2007 / IEC 62305-3:2006 |  |
|  | Protection against lightning Part 4: Electrical and electronic systems within structures - Provides information for the design, installation, inspection, maintenance and testing of a lightning electromagnetic impulse (LEMP) protection measures system (LPMS)for electrical and electronic systems within a structure, able to reduce the risk of permanent failures due to lightning electromagnetic impulse | SABS | SANS 62305-4:2007 / IEC 62305-4:2006 |  |
|  | Quality management systems - Guidelines for quality management in projects - Gives guidance on the application of quality management in projects. | SABS | SANS 274:2004 / ISO 10006:2003 (2009-04-17) |  |
|  | Decorative high gloss enamel paints - Specifies the requirements for two grades of air-drying gloss enamel paints for use on suitably primed and uncoated steel, wood, masonry, hardboard, compressed fibre board, and similar materials used in the construction and finishing of buildings. | SABS | SANS 630:2009 |  |
|  | Undercoats for paints - Specifies requirements for two grades of undercoat for use under air-drying paints and for application over primed steel and wood and on sealed and primed masonry, hardboard, compressed fibreboard and similar materials used in the construction and finishing of buildings. | SABS | SANS 681:2007 |  |
|  | Aluminium paint - This standard covers aluminium paint for use as a finishing coat on primed surfaces for exterior and interior exposure. This paint is suitable for use as a roof paint on suitably primed or previously painted galvanized iron. | SABS | SANS 682:2007 |  |
|  | Emulsion roof paint - Specifies requirements for roof paint based on synthetic polymers dispersed in a water phase. It makes provision for a self-priming paint for use on roofs, including new and aged galvanized steel roofs and sidings. | SABS | SANS 940:2005 (2010-06-17) |  |
|  | Pre-cast concrete paving slabs - Specifies the requirements for pre-cast concrete paving slabs for use as a wearing surface for pavements, footpaths, sidewalks, etc. but not for roads. | SABS | SANS 541:2004 (2010-03-19) |  |
|  | Plain-ended solid drawn copper tubes for potable water - Covers the requirements for plain-ended, solid drawn copper tubes for the conveyance of potable water. | SABS | SANS 460:2009 |  |
|  | Reinforced concrete pressure pipes - Covers requirements for materials, dimensions, joint types, cover to reinforcement, straightness and end squareness, hydraulic strength and proof load of reinforced concrete pipes of nominal internal diameter 100-1 800 mm. Includes information on special purpose pipes and a method for determining the amount of material in calcareous aggregate that is insoluble in hydrochloric acid. | SABS | SANS 676:2003 |  |
|  | Electric welded low carbon steel pipes for aqueous fluids (large bore) - Covers three grades of carbon steel pipes of size 219,1 mm to 2 230 mm used in the conveyance of water and other aqueous fluids. | SABS | SANS 719:2008 |  |
|  | Cast-iron pipes and pipe fittings for use above ground in drainage installations - Covers the requirements for low pressure cast-iron pipes and pipe fittings intended for use above ground as soil water, waste water, and vent pipes. | SABS | SANS 746:2010 |  |
|  | Fibre-cement pipes, couplings and fittings for sewerage, drainage and low-pressure irrigation - Covers uncoated fibre-cement (FC) pipes, couplings and fittings of nominal diameter not exceeding 1 000 mm, intended for use under conditions of gravity flow and atmospheric pressure sewerage, drainage and low-pressure irrigation applications. It covers both water-cured and autoclaved pipes, couplings and fittings that are manufactured essentially from an intimate mixture of hydraulic binders, asbestos (or other acceptable fibres)and water, with or without additions | SABS | SANS 819:2006 |  |
|  | Pitch-impregnated fibre pipes and fittings and jointing - Specifies the dimensional and minimum quality requirements for pitch-impregnated fibre pipes (known as pitch fibre pipes) of nominal diameter 75 mm, 100 mm and 150 mm, together with pitch fibre fittings and polypropylene and polyethylene couplings, that are suitable, when they have solid walls, for use as cable conduit and when they have perforated walls, for subsoil drainage. | SABS | SANS 921:2009 |  |
|  | Components of pressure pipe systems Part 1: Unplasticized poly(vinyl chloride) (PVC-U) pressure pipe systems - Specifies requirements for unplasticized poly(vinyl chloride) (PVC-U) pipes and injection-moulded fittings that are intended for above-ground and below-ground pressure applications for the conveyance of potable water in reticulation systems and for other applications in which continuous temperatures in excess of 25 °C are not encountered. It gives minimum wall thicknesses, based on a design stress rating of 10 MPa for pipes of nominal outside diameter 90 mm or less and 12,5 MPa for pipes of nominal outside diameter 110 mm and more. It covers seven classes of pipes and three classes of fittings for reticulation systems. It specifies 25 nominal sizes of pipes for reticulation systems (ranging from 16 mm to 630mm). | SABS | SANS 966-1:2010 |  |
|  | Components of pressure pipe systems Part 2: Modified poly(vinyl chloride)(PVC-M) pressure pipe systems - Specifies requirements for modified poly(vinyl chloride) (PVC-M) pipes (with integral joints that incorporate rubber sealing rings) and fittings (postformed from pipe made of PVC-M) that are intended for above-ground and below-ground pressure applications for the conveyance of potable water in reticulation systems and for other applications, in which continuous temperatures in excess of 25 °C are not encountered. It gives minimum wall thicknesses based on a design stress rating of 18 MPa for pipes of nominal outside diameter 50 mm to 630 mm. It covers six classes of pipe, together with post-formed fittings and bends. It includes requirements for the components of rubber joint rings. It does not cover fittings made by hot-gas and hot-plate fusion techniques. | SABS | SANS 966-2:2010 |  |
|  | Unplasticized poly(vinyl chloride) (PVC-U) soil, waste and vent pipes and pipe fittings - Covers unplasticized poly(vinyl chloride) (PVC-U) pipes (including vent pipes) and pipe fittings of nominal sizes 40 mm to 160 mm intended for above-ground non-pressure applications for the conveyance of soil (human excrement or faeces) and waste water where continuous temperatures in excess of 60 °C are not encountered. Covers fittings manufactured predominantly by the injection-moulding process, but does not cover fittings produced by fabrication only. Covers two methods of jointing. | SABS | SANS 967:2004 (2009-07-31) |  |
|  | Pre-stressed concrete pipes - Covers pre-stressed concrete non-cylinder pipes (and joints) intended primarily for the conveyance of water and sewage under pressure, but does not cover the fittings for these pipes. | SABS | SANS 975:2010 |  |
|  | Ductile iron pipes, fittings, accessories and their joints for water applications - Specifies the requirements and test methods applicable to ductile iron pipes, fittings, accessories and their joints for the construction of pipelines. | SABS | SANS 2531:2010 / ISO 2531:2009 |  |
|  | Plastics piping systems - Polyethylene (PE) pipes and fittings for water supply - Part 5: Fitness for purpose of the system - Specifies the characteristics of the fitness for purpose of assembled piping systems made from polyethylene (PE) intended for the conveyance of water for human consumption, including raw water prior to treatment and water for general purposes. | SABS | SANS 4427-5:2008 / ISO 4427-5:2007 |  |
|  | Polyethylene (PE) pipes for water supply - Specifications - Specifies the required properties of pipes made from polyethylene (PE) to be used for buried water mains and services and for water supply above ground both inside and outside buildings. In addition, it specifies some general properties of the material from which these pipes are made, including a classification scheme. | SABS | SANS 4427:1996 / ISO 4427:1996 (2009-07-31) |  |
|  | Plastics piping systems - Polyethylene (PE) pipes and fittings for water supply - Part 1: General -Specifies the general aspects of polyethylene (PE) piping systems (mains and service pipes) intended for the conveyance of water for human consumption, including raw water prior to treatment and water for general purposes. | SABS | SANS 4427-1:2008 / ISO 4427-1:2007 |  |
|  | Plastics piping systems - Polyethylene (PE) pipes and fittings for water supply - Part 2: Pipes - Specifies the pipes made from polyethylene (PE) intended for the conveyance of water for human consumption, including raw water prior to treatment and water for general purposes. | SABS | SANS 4427-2:2008 / ISO 4427-2:2007 |  |
|  | Plastics piping systems for non-pressure underground drainage and sewerage - Unplasticized poly(vinyl chloride) (PVC-U) - Specifies the requirements for unplasticized poly(vinyl chloride) (PVC-U) pipes, fittings and piping systems intended for use for non-pressure underground drainage and sewerage for the conveyance of soil and waste discharge of domestic and industrial origin, as well as surface water. | SABS | SANS 4435:2010 / ISO 4435:2003 |  |
|  | Rubber seals - Joint rings for water supply, drainage and sewerage pipelines - Specification for materials - Specifies requirements for materials used in vulcanized rubber seals for cold drinking-water supplies (up to 50 degrees Celsius), and drainage, sewerage and rainwater systems (continuous flow up to 45 degrees Celsius and intermittent flow up to 95 degrees Celsius). | SABS | SANS 4633:2008 / ISO 4633:2002 (2009-03-20) |  |
|  | Plastics piping systems for non-pressure underground drainage and sewerage - Polyethylene (PE) - Specifies the requirements for polyethylene (PE) pipes, fittings and piping systems intended for use for non-pressure underground drainage and sewerage for the conveyance of soil and waste discharge of domestic and industrial origin, as well as surface water. | SABS | SANS 8772:2010 / ISO 8772:2006 |  |
|  | Plastics piping systems for non-pressure underground drainage and sewerage - Polypropylene (PP) - Specifies the requirements for polypropylene (PP) pipes, fittings and piping systems intended for use for non-pressure underground drainage and sewerage for the conveyance of soil and waste discharge of domestic and industrial origin, as well as surface water. | SABS | SANS 8773:2007 / ISO 8773:2006 |  |
|  | Plastics piping systems - Polyethylene (PE) pipes for irrigation - Specifications - polyethylene (PE) intended to be used for the conveyance of water for irrigation. Specifies the pipes (mains, sub-mains and laterals) with nominal outside diameters from 12 mm up to and including 63 mm made from polyethylene (PE) intended to be used for the conveyance of water for irrigation. | SABS | SANS 8779:2010 / ISO 8779:2010 |  |
|  | Multilayer piping systems for hot and cold water installations inside buildings Part 1: General - Specifies the general aspects of multilayer piping systems intended to be used for hot and cold water installations within buildings for the conveyance of water whether or not intended for human consumption (domestic systems) and for heating systems, under design pressures and temperatures according to the class of application. | SABS | SANS 21003-1:2008 / ISO/FDIS 21003-1:2007 |  |
|  | Multilayer piping systems for hot and cold water installations inside buildings Part 2: Pipes - Specifies the characteristics of pipes for multilayer piping systems intended to be used for hot and cold water installations inside buildings for the conveyance of water whether or not intended for human consumption (domestic systems) and for heating systems, under design pressures and temperatures appropriate to the class of application. | SABS | SANS 21003-2:2008 / ISO/FDIS 21003-2:2007 |  |
|  | Multilayer piping systems for hot and cold water installations inside buildings Part 3: Fittings - Specifies the characteristics of fittings for multilayer piping systems intended to be used for hot and cold water installations inside buildings for the conveyance of water whether or not intended for human consumption (domestic systems) and for heating systems, under design pressures and temperatures appropriate to the class of application. | SABS | SANS 21003-3:2008 / ISO/FDIS 21003-3:2007 |  |
|  | Multilayer piping systems for hot and cold water installations inside buildings Part 5: Fitness for purpose of the system - Specifies the characteristics for the fitness for purpose of multilayer piping systems intended to be used for hot and cold water installations inside buildings for the conveyance of water whether or not intended for human consumption (domestic systems) and for heating systems, under design pressures and temperatures according to the class of application. | SABS | SANS 21003-5:2008 / ISO/FDIS 21003-5:2007 |  |
|  | Plastics piping systems for non-pressure underground drainage and sewerage - Structured-wall piping systems of unplasticized poly(vinyl chloride) (PVC-U), polypropylene (PP) and polyethylene (PE) Part 1: Material specifications and performance criteria for pipes, fittings and system - Specifies the definitions and requirements for pipes, fittings and the system based on unplasticized poly(vinyl chloride) (PVC-U), polypropylene (PP) and polyethylene (PE) structured-wall piping systems in the field of non-pressure underground systems for underground drainage and sewerage. | SABS | SANS 21138-1:2008 / ISO 21138-1:2007 |  |
|  | Plastics piping systems for non-pressure underground drainage and sewerage - Structured-wall piping systems of unplasticized poly(vinyl chloride) (PVC-U), polypropylene (PP) and polyethylene (PE) Part 2: Pipes and fittings with smooth external surface, Type A - Specifies definitions, requirements, test methods and test parameters for pipes with a smooth external surface (Type A), fittings and systems based on unplasticized poly(vinyl chloride) (PVC-U), polypropylene (PP) and polyethylene (PE) structured-wall piping in the field of non-pressure systems for underground drainage and sewerage. Gives recommendations concerning colours. Applies to pipes and fittings with or without an integral socket. | SABS | SANS 21138-2:2008 / ISO 21138-2:2007 |  |
|  | Plastics piping systems for non-pressure underground drainage and sewerage - Structured-wall piping systems of unplasticized poly(vinyl chloride) (PVC-U), polypropylene (PP) and polyethylene (PE) Part 3: Pipes and fittings with non-smooth external surface, Type B - Specifies the definitions and requirements for pipes with a non-smooth external surface (Type B), fittings and systems based on unplasticized poly(vinyl chloride) (PVC-U), polypropylene (PP) and polyethylene (PE) structured-wall piping systems in the field of non-pressure systems for underground drainage and sewerage. | SABS | SANS 21138-3:2008 / ISO 21138-3:2007 |  |
|  | Plastics piping systems for hot and cold water installations - Polyethylene of raised temperature resistance (PE-RT) Part 1: General - Specifies the general aspects of piping systems made of polyethylene of raised temperature resistance (PE-RT), intended to be used for hot and cold water installations within buildings for the conveyance of water - whether or not the water is intended for human consumption (domestic systems) or heating systems - under specific design pressures and temperatures according to the class of application. | SABS | SANS 22391-1:2008 / ISO 22391-1:2007 |  |
|  | Plastics piping systems for hot and cold water installations - Polyethylene of raised temperature resistance (PE-RT) Part 2: Pipes - Specifies the characteristics of pipe made from polyethylene of raised temperature resistance (PE-RT), intended to be used for hot and cold water installations within buildings for the conveyance of water - whether or not the water is intended for human consumption (domestic systems) or heating systems - under the design pressures and temperatures appropriate to the class of application. | SABS | SANS 22391-2:2008 / ISO 22391-2:2007 |  |
|  | Plastics piping systems for hot and cold water installations - Polyethylene of raised temperature resistance (PE-RT) Part 3: Fittings - Specifies the characteristics of fittings for piping systems made from polyethylene of raised temperature resistance (PE-RT), intended to be used for hot and cold water installations within buildings for the conveyance of water - whether or not the water is intended for human consumption (domestic systems) or heating systems - under the design pressures and temperatures appropriate to the class of application. | SABS | SANS 22391-3:2008 / ISO 22391-3:2007 |  |
|  | Plastics piping systems for hot and cold water installations - Polyethylene of raised temperature resistance (PE-RT) Part 5: Fitness for purpose of the system - Specifies the characteristics of the fitness for purpose of piping systems made from polyethylene of raised temperature resistance (PE-RT), intended to be used for hot and cold water installations within buildings for the conveyance of water - whether or not the water is intended for human consumption (domestic systems) or heating systems - under the design pressures and temperatures appropriate to the class of application. | SABS | SANS 22391-5:2008 / ISO 22391-5:2007 |  |
|  | Plastics piping systems - Polyethylene (PE) pipes and fittings for water supply - Part 3: Fittings- Specifies the general aspects of fittings made from polyethylene (PE) for piping systems intended for the conveyance of water for human consumption, including raw water prior to treatment and water for general purposes. | SABS | SANS 4427-3:2008 / ISO 4427-3:2007 |  |
|  | Vitrified clay pipes and fittings and pipe joints for drains and sewers Part 1: Requirements - Specifies requirements for flexibly jointed vitrified clay pipes and fittings with or without sockets for the construction of drainage and sewerage systems. The preferred dimensions for pipe lengths, curvature of bends and angles of junction arms are specified. | SABS | SANS 50295-1:1991 / EN 295-1:1991 (2007-06-01) |  |
|  | Vitrified clay pipes and fittings and pipe joints for drains and sewers Part 2: Quality control and sampling - Specifies requirements for manufacturer's internal quality control, third party assessment and inspection. | SABS | SANS 50295-2:1991 / EN 295-2:1991 (2007-05-18) |  |
|  | Vitrified clay pipes and fittings and pipe joints for drains and sewers Part 3: Test methods - Specifies the test methods required for EN 295-1. Reference to pipes includes pipe sections as defined in EN 295-1 where these are suitable and permitted for the tests. | SABS | SANS 50295-3:1991 / EN 295-3:1991 (2007-06-01) |  |
|  | Ductile iron pipes, fittings, accessories and their joints for water pipelines - Requirements and test methods - Specifies the requirements and associated test methods applicable to ductile iron pipes, fittings, accessories and their joints for the construction of pipelines to convey water (e.g. potable water), with or without pressure, and to be installed below or above ground. Specifies requirements for materials, dimensions and tolerances, mechanical properties and standard coatings of ductile iron pipes and fittings. | SABS | SANS 50545:2003 / EN 545:2002 (2009-12-11) |  |
|  | Plastics piping systems - Plastics pipes and fittings - Determination of the opacity - Specifies a method for determining the opacity of plastics pipes and fittings. | SABS | SANS 50578:1993 / EN 578:1993 |  |
|  | Ductile iron pipes, fittings, accessories and their joints for sewerage application - Requirements and test methods - Specifies the requirements and associated test methods applicable to ductile iron pipes, fittings, accessories and their joints for the construction of drains and sewers outside buildings: operating without pressure, or with positive or negative pressure, installed below or above ground, and for conveyance of surface water, domestic waste water and certain types of industrial effluents, either in separate systems or in mixed systems. | SABS | SANS 50598:1994 / EN 598:1994 (2009-12-11) |  |
|  | Elasto-metric seals - Material requirements for pipe joint seals used in water and drainage applications Part 1: Vulcanized rubber - Specifies requirements for materials used in vulcanized rubber seals for cold potable water supply (up to 50 °C), hot potable and non-potable water supply (up to 110 °C), and drainage, sewerage and rainwater systems (continuous flow up to 45 °C and intermittent flow up to 95 °C). The different designations of seals specified are defined according to their type, application and requirements. General requirements for finished joint seals are also given; any additional requirements called for by the particular application are specified in the relevant product standards taking into account that the performance of pipe joints is a function of the seal material properties, seal geometry and pipe joint design. | SABS | SANS 50681-1:1996 / EN 681-1:1996 (2008-07-25) |  |
|  | Plastics piping systems - Mechanical joints between fittings and polyolefin pressure pipes - Test method for leak tightness under internal pressure of assemblies subjected to bending - Specifies a method for checking the leak tightness under internal hydrostatic pressure of assembled mechanical joints (excluding fusion-welded joints) between fittings and polyolefin pressure pipes while subjected to bending. Details a calculation method for the average bending radius and the procedure for achieving this bending. Is applicable regardless of the design and material of the fitting used for jointing pipes. | SABS | SANS 50713:1993 / EN 713:1993 (2007-02-23) |  |
|  | Pipes and fittings of longitudinally welded stainless steel pipes with spigot and socket for waste water systems Part 1: Requirements, testing, quality control - Specifies requirements, tests and quality control for longitudinally welded, stainless steel pipes and fittings with spigot and socket for use in water systems usually operating under gravity or at a low head of pressure. | SABS | SANS 51124-1:2008 / EN 1124-1:1999 |  |
|  | Pipes and fittings of longitudinally welded stainless steel pipes with spigot and socket for waste water systems Part 2: System S -Dimensions - Applies to pipes and fittings of longitudinally welded stainless steel pipes with spigot and socket for waste water systems. Specifies dimensions and tolerances for pipes, fittings and pipe connectors and establishes a system of designations for the different pipe and pipe fitting types that conform to the stated requirements. | SABS | SANS 51124-2:2008 / EN 1124-2:2007 |  |
|  | Pipes and fittings of longitudinally welded stainless steel pipes with spigot and socket for waste water systems Part 3: System X - Dimensions - Applies to pipes and fittings of longitudinally welded stainless steel pipes with spigot and socket for waste water systems. It specifies dimensions, tolerances and seals of System X. | SABS | SANS 51124-3:2008 / EN 1124-3:1999 |  |
|  | Ductile iron fittings for PVC-U or PE piping systems - Requirements and test methods - Specifies the requirements and associated test methods applicable to ductile iron fittings and their joints to be used with poly(vinyl chloride) (PVCU) pipes or polyethylene (PE) pipes, for the construction of pipelines and to convey water (e.g. potable water) with or without pressure. | SABS | SANS 52842:2007 / EN 12842:2000 |  |
|  | Steel mesh reinforced polyethylene (PE) pipes for water supply - Specifies the required properties of steel mesh reinforced polyethylene (PE) pipes made by combining mesh-shaped steel reinforcement with polyethylene through extrusion technology. The requirements for raw materials, marking, packing, storage and handling of the pipes are also specified. The steel mesh is made by continuously winding and welding transverse wires spirally to the longitudinal wires. | SABS | SANS 370:2009 |  |
|  | Steel mesh reinforced polyethylene (PE) pipe fittings for water supply - Specifies the required properties, classification and geometrical characteristics of steel mesh reinforced polyethylene (PE) pipe fittings made by combining mesh-shaped steel reinforcement with polyethylene through injection moulding technology. The requirements for raw materials, marking, packing, storage and handling of the pipe fittings are also specified. The steel mesh is made by winding and welding perforated steel plate to a tube. Applies to steel mesh reinforced polyethylene (PE) pipe fittings intended to be used for the conveyance of water under pressure, for general purposes, as well as for the supply of drinking water, with temperatures not exceeding 80 °C. | SABS | SANS 371:2009 |  |
|  | Bending dimensions and scheduling of steel reinforcement for concrete - Specifies the bending dimensions (including the methods of measurement) and the scheduling of steel bars for use in the reinforcement of concrete. | SABS | SANS 282:2004 |  |
|  | Steel bars for concrete reinforcement - Covers the physical and mechanical requirements for carbon steel bars intended for use as reinforcement for concrete. | SABS | SANS 920:2005 |  |
|  | Road markings Part 1: Single-pack solvent-borne paints - This part of SABS 731 specifies requirements for conventional solvent-borne paints suitable for marking traffic-bearing bituminous or concrete road and runway surfaces, and makes provision for white, yellow and other colours. | SABS | SANS 731-1:2006 |  |
|  | Road markings Part 1: Single-pack solvent-borne paints - This part of SABS 731 specifies requirements for conventional solvent-borne paints suitable for marking traffic-bearing bituminous or concrete road and runway surfaces, and makes provision for white, yellow and other colours. | SABS | SANS 731-1:2006 |  |
|  | Road markings Part 2: Single-pack water-borne paints - Specifies requirements for conventional water-borne paints suitable for marking traffic-bearing bituminous or concrete road surfaces, and makes provision for white, yellow and other colours. | SABS | SANS 731-2:2006 |  |
|  | Road markings Part 2: Single-pack water-borne paints - Specifies requirements for conventional water-borne paints suitable for marking traffic-bearing bituminous or concrete road surfaces, and makes provision for white, yellow and other colours. | SABS | SANS 731-2:2006 |  |
|  | Road tar binders prepared from Lurgi-gasification crudes - Covers the performance requirements for three types (available in a total of eight viscosity grades) of road tar binders prepared entirely from crude tars produced from the Lurgi-gasification process, except that a Grade RTL 3/12 P primer may be manufactured from crudes other than those obtained from the Lurgi process | SABS | SANS 749:2003 |  |
|  | Road tar binders prepared from Lurgi-gasification crudes - Covers the performance requirements for three types (available in a total of eight viscosity grades) of road tar binders prepared entirely from crude tars produced from the Lurgi-gasification process. | SABS | SANS 749:2003 |  |
|  | Pre-cast concrete kerbs, edgings and channels - Specifies the dimensional and physical requirements for various types of pre-cast concrete kerbs, edgings and channels intended for use in the construction of roads and footpaths. | SABS | SANS 927:2007 |  |
|  | Rubber, vulcanized - Preformed compression seals for use between concrete motorway paving sections - Specification for material - Specifies requirements for material for pre-formed vulcanized rubber seals used between concrete motorway paving sections. | SABS | SANS 4635:2004 / ISO 4635:1982 (2009-03-20) |  |
|  | Rubber, vulcanized - Preformed compression seals for use between concrete motorway paving sections - Specification for material - Specifies requirements for material for pre-formed vulcanized rubber seals used between concrete motorway paving sections. | SABS | SANS 4635:2004 / ISO 4635:1982 (2009-03-20) |  |
|  | Road marking materials - Drop on materials - Glass beads, antiskid aggregates and mixtures of the two - Specifies the requirements for laboratory tests (production control) and qualification procedures for the drop on materials used in road marking. | SABS | SANS 51423:1997 / EN 1423:1997 (2007-03-23) |  |
|  | Road marking materials - Premix glass beads - Specifies the requirements for laboratory tests (production control) and qualification procedures for the premixed glass beads used in road marking materials. The requirements taken into consideration are granulometry, refractive index of the glass, chemical resistance, quality and surface treatments. | SABS | SANS 51424:1997 / EN 1424:1997 (2007-03-23) |  |
|  | Mastic asphalt for roofing - Covers mastic asphalt for roofing, other than asphalt designed to withstand the exceptional conditions experienced in ammunition factories, etc., and direct attack by industrial liquors. | SABS | SANS 297:2009 |  |
|  | Mastic asphalt for damp-proof courses and tanking - Covers two types of mastic asphalt for damp-proof courses and tanking (horizontal DPC and vertical DPC tanking), other than mastic asphalt designed to withstand direct attack by industrial liquors. | SABS | SANS 298:2007 |  |
|  | Chloroprene rubber sheet (for waterproofing) - Covers chloroprene rubber sheet used in waterproofing applications. Requirements for materials, workmanship, dimensions and physical and chemical properties are specified. | SABS | SANS 580:2007 |  |
|  | Fibre-cement sheets (flat and profiled) - Specifies requirements for flat and profiled (straight and curved) sheets manufactured from fibre-cement. It does not cover coatings that are applied after the manufacture of the sheets. | SABS | SANS 685:2006 |  |
|  | Aluminium alloy corrugated and troughed sheets - Covers 4 types of straight corrugated and 6 types of straight troughed aluminium alloy sheets. Requirements are given for the chemical composition, tensile strength, resistance to bending, thickness, number of corrugation or pitches, width, length, squareness and finish. An appendix describes recommended practices to be observed when these sheets are used. | SABS | SANS 903:2010 |  |
|  | Cast iron surface boxes and manhole and inspection covers and frames - Covers the material, dimensional, mass, constructional, and strength requirements for fifteen types of cast iron manhole covers and frames, eleven types of cast iron surface boxes (with covers) for meters, valves, stopcocks, and hydrants, and two types of cast iron inspection covers and frames. | SABS | SANS 558:2009 |  |
|  | Vitrified clay sewer pipes and fittings - Covers vitrified clay sewer pipes and fittings suitable for jointing by one of the following methods: mortar caulking jointing, rolling rubber ring jointing, bitumen run jointing, preformed factory-applied jointing, plain-end pipe jointing, or slip-seal rubber jointing. | SABS | SANS 559:2005 |  |
|  | Steel-reinforced spirally wound PE drainage and sewer pipes - Specifies requirements and inspection methods for steel-reinforced spirally-wound polyethylene (PE) drainage pipes made by winding and welding composite profiles, intended to be used for municipal drainage pipelines, municipal sewers, industrial drainage pipelines, industrial sewers, as well as agricultural irrigation systems | SABS | SANS 674:2010 |  |
|  | Gully tops and manhole tops for vehicular and pedestrian areas - Design requirements, type testing, marking, quality control - Applies to gully tops and manhole tops with a clear opening up to and including 1 000 mm, for installation within areas subjected to pedestrian or vehicular traffic (or both). Does not apply to surface boxes nor to floor and roof gullies in buildings which are specified in prEN 1253. Establishes definitions, classes, materials, design and testing requirements, marking and quality control of gully tops and manhole tops. | SABS | SANS 50124:1994 / EN 124:1994 (2009-12-11) |  |
|  | Gullies for buildings Part 1: Requirements - Specifies gullies, gives guidance for places of installation and requirements for the construction, design, performance and marking of factory made gullies for buildings, irrespective of material, for use in drainage systems operating under gravity including siphonic systems. | SABS | SANS 51253-1:2010 / EN 1253-1:2003 |  |
|  | Gullies for buildings Part 2: Test methods - Specifies test methods for gullies for buildings according to EN 1253-1:2003 and access covers in accordance with EN 1253-4 (published in South Africa as an identical adoption under the designation SANS 51253-1 and SANS 51253-4). | SABS | SANS 51253-2:2010 / EN 1253-2:2003 |  |
|  | Gullies for buildings Part 3: Quality control - Specifies the requirements for quality control for gullies and access covers for buildings to ensure conformity of these products with EN 1253-1:1999 and EN 1253-4 (published in South Africa as an identical adoption under the designation SANS 51253-1 and SANS 51253-4). | SABS | SANS 51253-3:2010 / EN 1253-3:1999 |  |
|  | Gullies for buildings Part 4: Access covers - Specifies access covers according to their loading strength and requirements relating to their design, construction, marking, testing and quality control. | SABS | SANS 51253-4:2010 / EN 1253-4:1999 |  |
|  | Gullies for buildings Part 5: Gullies with light liquids closure - Specifies requirements for the design, construction, performance, application and marking as well as test methods of factory made gullies with a light liquid closure for buildings. | SABS | SANS 51253-5:2010 / EN 1253-5:2003 |  |
|  | Gravity drainage systems inside buildings Part 1: General and performance requirements - Specifies waste water drainage systems which operate under gravity and is applicable for drainage systems inside dwellings and commercial, institutional and industrial buildings. | SABS | SANS 52056-1:2010 / EN 12056-1:2000 |  |
|  | Gravity drainage systems inside buildings Part 2: Sanitary pipework, layout and calculation - Specifies waste water drainage systems which operate under gravity and is applicable for drainage systems inside dwellings and commercial, institutional and industrial buildings. | SABS | SANS 52056-2:2010 / EN 12056-2:2000 |  |
|  | Gravity drainage systems inside buildings Part 3: Roof drainage, layout and calculation - Specifies wastewater drainage systems, which operate under gravity and is applicable for drainage systems inside dwellings and commercial, institutional and industrial buildings. | SABS | SANS 52056-3:2010 / EN 12056-3:2000 |  |
|  | Gravity drainage systems inside buildings Part 4: Wastewater lifting plants - Layout and calculation - Specifies layout, operation and maintenance requirements for lifting plants for wastewater containing faecal matter, faecal-free wastewater and rainwater within buildings and sites, together with their discharge pipework and connection to drain and faecal wastewater lifting plants for limited applications | SABS | SANS 52056-4:2010 / EN 12056-4:2000 | P670 |
|  | Gravity drainage systems inside buildings Part 5: Installation and testing, instructions for operation, maintenance and use - Specifies waste water drainage systems which operate under gravity and is applicable for drainage systems inside dwellings and commercial, institutional and industrial buildings. | SABS | SANS 52056-5:2010 / EN 12056-5:2000 |  |
|  | Small wastewater treatment systems for up to 50 PT Part 1: Prefabricated septic tanks - Specifies the requirements for prefabricated septic tanks for the partial treatment of domestic wastewater for a population not exceeding fifty. | SABS | SANS 52566-1:2004 / EN 12566-1:2000 |  |
|  | Internal and/or external protective coatings for steel tubes - Specification for hot dip galvanized coatings applied in automatic plants - Specifies the requirements and tests for hot dip galvanized coatings applied to steel tubes hot dip galvanized in automatic plants for gas and water installations, including water intended for human consumption and other applications, e.g. scaffold tubes and structural hollow sections. | SABS | SANS 32:1997 / EN 10240:1997 (2004-10-15) |  |
|  | Light steel frame building - Specifies requirements for the design, fabrication and construction of buildings with light steel frames, clad and insulated with appropriate materials, including the walls, roofs, floors, and foundations of such buildings | SABS | SANS 517:2009 |  |
|  | Cathodic protection measurement techniques - Deals with the cathodic protection against corrosion of buried or immersed metallic structures, detailing the measuring methods to be used for assessing the effectiveness of cathodic protection as well as the measurements and measures taken to monitor cathodic protection during operation. | SABS | SANS 53509:2009 / EN 13509:2003 |  |
|  | Wooden poles, droppers, guardrail posts and spacer blocks Part 2: Softwood species - Specifies requirements for preservative-treated softwood poles, droppers, guardrail posts and spacer blocks. | SABS | SANS 457-2:2008 |  |
|  | Wooden poles, droppers, guardrail posts and spacer blocks Part 3: Hardwood species - Specifies requirements for preservative-treated hardwood structural poles, agricultural poles, fencing poles, round droppers, guardrail posts and spacer blocks. | SABS | SANS 457-3:2000 |  |
|  | Timber preservative - Alkaline-copper-quaternary type - Covers alkaline-copper-quaternary (ACQ) type preservatives for wood. | SABS | SANS 506:2008 |  |
|  | Wood preservative mixtures containing 1) high temperature creosote and coal tar or 2) low and medium temperature creosote and coal tar (SANS 592 and SANS 593, bound together) - Covers a type D wood preservative mixture containing creosote and coal tar. It is a low and medium temperature creosote, containing up to 40 per cent of coal tar expressed as a percentage of the total volume of the mixture. | SABS | SANS 593 AND SANS 592:1957 |  |
|  | Wood-preserving creosote mixtures and coal-tar mixtures - Covers coal tar that is derived entirely from the carbonization of bituminous coal; this coal tar may either be a coal-tar distillate (creosote) or a solution of coal tar in coal-tar distillate and is intended for use in the preservation of timber. | SABS | SANS 616:2008 |  |
|  | Primers for wood - This standard covers primers for wood, for interior and exterior use. | SABS | SANS 678:2005 |  |
|  | Timber structures - Product requirements for prefabricated structural members assembled with punched metal plate fasteners - Specifies product requirements for prefabricated structural members (e.g. trusses, beams and girders) for use in buildings and bridges made from members of structural timber (with or without finger joints) assembled with punched metal plate fasteners. | SABS | SANS 54250:2010 / EN 14250:2004 |  |
|  | Water taps (metallic bodies) - Covers requirements for four classes of screw-down and non-screw-down metallic water taps and stopcocks of sizes up to and including 50 mm for the supply of water at temperatures not exceeding 75 °C. | SABS | SANS 226:2009 |  |
|  | Wedge gate and resilient seal valves for waterworks Part 1: General - Covers the general requirements for the construction and performance of flanged, spigotted and socket-ended wedge and resilient seal valves of the rising and non-rising spindle configuration of waterworks. | SABS | SANS 664-1:2009 |  |
|  | Wedge gate and resilient seal valves for waterworks Part 2: Wedge gate valves - Covers the requirements for the design and construction of wedge gate valves of pressure rating (PN) up to 4 000 kPa and of sizes up to 1 000mm. | SABS | SANS 664-2:2009 |  |
|  | Wedge gate and resilient seal valves for waterworks Part Part 3: Resilient seal valves - Covers the requirements for the design and construction of resilient seal valves of pressure rating (PN) up to 2 500 kPa and of sizes up to 600 m | SABS | SANS 664-3:2009 |  |
|  | Float valves - Covers the requirements for two classes, two designs and three types of float valve of nominal size not exceeding 50 mm that are intended for use with water at a temperature not exceeding 50 °C. It does not cover the requirements for floats. | SABS | SANS 752:2009 |  |
|  | Fibre-cement boards - Specifies the requirements for smooth surface and textured surface fibre-cement boards. | SABS | SANS 803:2005 |  |
|  | Specification for masonry units Part 4: Autoclaved aerated concrete masonry units - Specifies the characteristics and performance requirements of autoclaved aerated concrete (AAC) masonry units for which the main intended uses are different types of load bearing and non-load bearing applications in all forms of walling including single leaf, cavity, partitions, retaining, basement and general use below ground level, including walling for fire protection, thermal insulation, sound insulation and the fabric of chimneys (excluding chimney flue units).It defines the performance related to e.g. strength, density, dimensional accuracy etc. and provides for the evaluation of conformity of the product to this standard. The marking requirement for products covered by this standard is included. | SABS | SANS 50771-4:2007 / EN 771-4:2003 |  |
|  | Mobile waste containers Part 1: Containers with 2 wheels with a capacity up to 400 l for comb lifting devices, dimensions and design - Specifies dimensions and design requirements of mobile waste containers with 2 wheels, with capacity up to 400 l to be used by comb lifting devices. | SABS | SANS 95-1:2009 / EN 840-1:2004 |  |
|  | Mobile waste containers Part 2: Containers with 4 wheels with a capacity up to 1 300 l with flat lid(s), for trunnion and/or comb lifting devices - Dimensions and design - Specifies dimensions and design requirements of mobile waste containers with 4 wheels, with flat lid(s) and capacity up to 1 300 l to be used by trunnion and/or comb lifting devices | SABS | SANS 95-2:2009 / EN 840-2:2004 |  |
|  | Steel refuse bins - Covers requirements for the dimensions, construction and strength of zinc-coated steel bins of nominal capacities 85 L and 57 L. | SABS | SANS 493:2009 |  |
|  | Water quality - Enumeration of culturable micro-organisms - Colony count by inoculation in a nutrient agar culture medium - Specifies a method for the enumeration of culturable micro-organisms in water by counting the colonies formed in a nutrient agar culture medium after aerobic incubation at 36 °C and 22 °C. The method is intended to measure the operational efficiency of the treatment process of public drinking water supplies and for general application to all types of water. It is particularly applicable to the examination of water intended for human consumption, including water in closed containers and to natural mineral waters. | SABS | SANS 313:2004 / ISO 6222:1999 (2010-08-13) |  |
|  | Sorbents for hydrocarbon-based products - Covers various types of sorbent materials used to remove hydrocarbon-based liquids from water, soil, and hard surfaces; these include absorbents which are designed to readily release collected liquids under pressure as well as those that are designed to retain it, as well as adsorbents. | SABS | SANS 339:2007 |  |
|  | Standard test methods for chloride ion in water - Specifies test methods that cover the determination of chloride ion in water, wastewater and brines. | SABS | SANS 374:2005 / ASTM D 512:2004 |  |
|  | Standard test method for turbidity of water - Specifies a test method that covers the determination of turbidity in water and wastewater. The method is applicable to the measurement of turbidities in the range from 1,0 to 40 nephelometric turbidity units (NTU). | SABS | SANS 375:2005 / ASTM D 1889:2000 |  |
|  | Water quality - Determination of calcium content - EDTA titrimetric method - Specifies a titrimetric method using ethylenediaminetetraacetic acid (EDTA) for the determination of the calcium content of ground waters, surface waters and drinking waters. It can also be used for municipal and industrial raw waters, provided they do not contain interfering amounts of heavy metals. It is applicable to waters with calcium contents ranging from 2 mg/L to 100 mg/L (0,05 mmol/L to 2,5 mmol/L). | SABS | SANS 450:2005 / ISO 6058:1984 |  |
|  | Rubber insertion sheeting. - Covers fabric-reinforced rubber sheeting used as jointing in low pressure water, air or steam lines. Dimensions, construction, physical and aging properties are specified. | SABS | SANS 564:2007 |  |
|  | Water quality - Sampling Part 13: Guidance on sampling of sludges from sewage and water treatment works - Gives guidance on the sampling of sludges from wastewater treatment works, water treatment works and industrial processes. Is applicable to all types of sludge arising from these works and also to sludges of similar characteristics, for example septic tank sludges. | SABS | SANS 5667-13:2007 / ISO 5667-13:1997 |  |
|  | Water quality - Enumeration of micro-organisms in water samples - Guidance on the estimation of variation of results with particular reference to the contribution of uncertainty of measurement - Gives guidance on interpretation of the requirements in BS EN ISO/IEC 17025:2005 (published in South Africa as an identical adoption under the designation SANS 17025:2005) with regard to the uncertainty of measurement in microbial counts in water samples. It gives guidance on the estimation of variation in results between replicate sub-samples, which will include that due to uncertainty of measurement, in order to assess whether the variation due to uncertainty of measurement has been controlled in accordance with the requirements specified in BS EN ISO/IEC 17025:2005. It is applicable to examination of sub-samples as part of a quality assurance system. It is applicable to samples of all types of water | SABS | SANS 886:2009 / BS 8496:2007 |  |
|  | Water quality - Sampling Part 5: Guidance on sampling of drinking water from treatment works and piped distribution systems - Establishes principles to be applied to the techniques of sampling water intended for human consumption, where water is drawn from municipal or similar distribution systems (including individual systems) where either prior treatment or quality assessment (or both) has resulted in the water being classified as suitable for drinking or potable process purposes. It is specifically applicable to water that is in continuous supply relative to any stage of use up to and including the point of consumption in a distribution system. This includes distribution within large buildings in which additional water quality management might be applicable. It is also applicable to sampling situations that can arise relative to the investigation of system defects or emergency situations where the safety of sampling operatives is not compromised. | SABS | SANS 5667-5:2006 / ISO 5667-5:2006 |  |
|  | Glass fibre reinforced plastics cisterns for cold-water storage Part 2: Specification for one-piece cisterns of nominal capacity from 500 l to 25 000 l - Specifies requirements for one-piece glass fibre reinforced plastics cold water storage cisterns, combined feed and expansion cisterns and expansion cisterns and their covers, for both domestic and industrial use. Particular requirements for additional components for cisterns for domestic use are also included. The range of capacities covered is from 500 l to 25 000 l. | SABS | SANS 7491-2:1992 / BS 7491-2:1992 |  |
|  | Glass fibre reinforced plastics cisterns for cold water storage Part 3: Specification for sectional tanks - Specifies requirements for rectangular glass fibre reinforced plastics (GRP) sectional tanks, constructed from square or rectangular panels, for storage of cold water for both domestic and industrial use. The water is in contact with the GRP tank surface and the working pressures do not exceed those produced by the static heads of water when the tanks are filled to their maximum working depths. | SABS | SANS 7491-3:1994 / BS 7491-3:1994 |  |
|  | Products used for treatment of water intended for human consumption - Inorganic supporting and filtering materials - Definitions - Applicable to all inorganic supporting and filtering materials (ISFM) used for treatment of water intended for human consumption. Defines terms relating to ISFM. | SABS | SANS 52901:2007 / EN 12901:1999 |  |
|  | Products used for treatment of water intended for human consumption - Inorganic supporting and filtering materials - Methods of test - Specifies methods of test to determine physical and chemical properties of Inorganic Supporting and Filtering Materials (ISFM). | SABS | SANS 52902:2007 / EN 12902:2004 |  |
|  | Products used for treatment of water intended for human consumption - Powdered activated carbon - Applicable to powdered activated carbon used for treatment of water intended for human consumption. Describes the characteristics of powdered activated carbon and specifies the requirements and the corresponding test methods for powdered activated carbon. It also gives information on its use in water treatment. | SABS | SANS 52903:2007 / EN 12903:2003 |  |
|  | Products used for treatment of water intended for human consumption - Silica sand and silica gravel - Applicable to silica sand and silica gravel used for treatment of water intended for human consumption. Describes the characteristics of silica sand and silica gravel and specifies the requirements and the corresponding test methods for silica sand and silica gravel. It also gives information on their use in water treatment. | SABS | SANS 52904:2007 / EN 12904:2005 |  |
|  | Products used for the treatment of water intended for human consumption - Granular activated carbon Part 1: Virgin granular activated carbon - Gives requirements for specification and acceptance conditions of raw materials for GRP tanks and vessels with or without lining for storage or processing of fluids, factory made or site built, non pressurised or pressurised, for use above ground. | SABS | SANS 52915-1:2008 / EN 12915-1:2003 |  |
|  | Products used for the treatment of water intended for human consumption - Granular activated carbon Part 2: Reactivated granular activated carbon - Applicable to reactivated granular activated carbon used for treatment of water intended for human consumption. Describes the characteristics of reactivated granular activated carbon and specifies the requirements and the corresponding test methods for reactivated granular activated carbon. It also gives information on its use in water treatment. | SABS | SANS 52915-2:2008 / EN 12915-2:2003 |  |
|  | GRP tanks and vessels for use above ground Part 1: Raw materials - Specification conditions and acceptance conditions - Applicable to virgin granular activated carbon used for treatment of water intended for human consumption. Describes the characteristics of virgin granular activated carbon and specifies the requirements and the corresponding test methods for virgin granular activated carbon. It also gives information on its use in water treatment. | SABS | SANS 53121-1:2009 / EN 13121-1:2003 |  |
|  | Specification for glass fibre reinforced cisterns of one-piece and sectional construction, for the storage, above ground, of cold water - Specifies the requirements for rectangular and vertical cylindrical glass reinforced plastics (GRP) one-piece cisterns and rectangular sectional tanks for the storage of water above ground for both domestic and industrial use within the following capacity ranges: a) one-piece cistern, 500 L to 100000 L; b) sectional tank, 500 L and larger. Particular requirements for additional components for cisterns for domestic use are also included. | SABS | SANS 53280:2009 / EN 13280:2001 |  |
|  | Products used for treatment of water intended for human consumption - Bentonite - Applicable to bentonite used for treatment of water intended for human consumption. Describes the characteristics of bentonite and specifies the requirements and the corresponding test methods for bentonite. It also gives information on its use in water treatment. | SABS | SANS 53754:2008 / EN 13754:2003 |  |
|  | Filament-wound FRP pressure vessels - Materials, design, manufacturing and testing - Specifies the requirements for the design including raw materials, calculation, manufacturing including composite materials, and testing of seamless Glass Reinforced Plastic (GRP) pressure vessels with protective layer, using only multi-directional filament winding, made in a factory and for use above ground and for storage and processing of fluids. | SABS | SANS 53923:2009 / EN 53923:2005 |  |
|  | Welding consumables - Rods, wires and deposits for tungsten inert gas welding of non-alloy and fine-grain steels - Classification - Specifies requirements for classification of rods and wires in the as-welded condition and in the post-weld heat-treated condition for tungsten inert gas welding of non-alloy and fine grain steels with a minimum yield strength of up to 500 MPa or a minimum tensile strength of up to 570 MPa. | SABS | SANS 53:2005 / ISO 636:2004 |  |
|  | Destructive tests on welds in metallic materials - Bend tests - Specifies a method for making transverse root, face and side bend tests on test specimens taken from butt welds, butt welds with cladding and cladding without butt welds in order to assess ductility or absence of imperfections (or both) on or near to the surface of the joint. Specifies a method for making longitudinal root and face bends tests to be used instead of transverse bend tests for heterogeneous assemblies when base materials or filler metal (or both) have a significant difference in their physical and mechanical properties in relation to bending. Applies to metallic materials in all forms of product with welded joints made by any fusion arc welding process. | SABS | SANS 547:2006 / ISO 5173:2000 |  |
|  | Power sources for manual arc welding with limited duty - Applies to welding power sources for manual metal arc welding with limited duty with covered stick electrodes. These welding power sources are limited to a rated maximum welding current of 160 A, are fitted with a thermal cut-out device, have performance based on the number of reference electrodes, capable of being melted with the welding power source in the cold and hot state and carry on their rating plate a declaration of the fuse size necessary for the maximum output. | SABS | SANS 50060:1989 / EN 50060:1989 |  |
|  | Welding consumables - Covered electrodes for manual metal arc welding of stainless and heat resisting steels - Classification - Specifies requirements for classification of covered electrodes based on the all-weld metal in the as welded or heat treated conditions for manual metal arc welding of stainless and heat resisting steels. | SABS | SANS 51600:1997 / EN 1600:1997 (2007-03-30) |  |
|  | Thermoplastics piping systems - End-load bearing mechanical joints between pressure pipes and fittings - Test method for resistance to pull-out under constant longitudinal force - Specifies a method for checking the ability of assembled end-load bearing mechanical joints (excluding fusion-welded joints) between fittings and pressure pipes to withstand a longitudinal tensile force. | SABS | SANS 50712:1993 / EN 712:1993 (2007-02-23) |  |
|  | Mixing water for concrete - Specification for sampling, testing and assessing the suitability of water, including water recovered from processes in the concrete industry, as mixing water for concrete - Specifies the requirements for water that is suitable for making concrete that conforms to EN 206-1 and describes methods for assessing its suitability. | SABS | SANS 51008:2006 / EN 1008:2002 |  |
|  | Site layout plan directive for Consultants |  |  | Attached |
|  | Final Account guideline |  |  | Attached |

SITE LAYOUT PLAN:  DIRECTIVE TO THE CONSULTANT

The Directive provides minimum requirements for the preparation of a Site Layout Plan and is based on the specifications of the Department of Public Works.

Specifications

The Directive is based on the following specifications:

Civil Engineering Manual PW347

Specification of Materials and Methods to be Used PW371

Additional Specification SB:  Operating and Maintenance Manuals.

Detail Ground Survey:

All services must be shown on a complete Site Layout Plan as required by the Engineer, including roads, fences, paving, transmission and telephone lines, etc.  For sewerage reticulation and stormwater drainage systems the pipe sizes, as well as invert heights must be provided.  An effort must be made to trace the routes of these services.

- Survey of Buildings

The “footprint” of all the buildings and structures must be surveyed.

- General

All survey data shall be captured in electronic format.

Title block

The standard drawing sheet layout and title block as specified by the Department of Public Works must be used.

Complete all the relevant fields in the title block with reference to the name of the port of entry in the appropriate block.  The words SITE LAYOUT PLAN shall form part of the drawing title.

Drawing title

The drawing number shall consist of a three-part identifier:

Port of entry designator	:	WCS 045305

Drawing number		:	Numbering will start at 1

Revision number		:	Will start at 01

Typical example:	WCS 045305/1 Rev 01

Overlay sheets/Layering scheme (if required)

The overlay sheet designator identifies the type of drawing (example:  overlay for water reticulation) and can be added to the drawing number:

C:	Existing structures, facilities, roads, paving, fencing, etc

CR:	Stormwater drainage system

CE:	Electrical power and equipment

CF:	Fire fighting equipment

CS:	Sewer network

CT:	Telephone lines

CW:	Water reticulation system

Typical example for the numbering of an overlay sheet:

WCS 045305/1CW Rev 01

Drafting conventions

Drafting conventions

The Site Layout Plan shall be created by following engineering conventions and standards in order to represent a clear drawing simplifying the huge amount of visual information:

Paper prints

Preference is given to size A1 plans, but for reporting size A3 will be used and the information shall still be legible in this format.

Scale

The Site Layout Plan shall be drawn according to scale. The following scales shall be used in order to fit on an A1 sheet (see “key layout” if a port of entry is too large to fit on one sheet):

1:200 or

1:500 or

1:1000

Plan orientation

The port of entry shall be rotated on the plan so that the north point arrow are pointing in the direction of either the upper left or upper right quadrants of the plan.  The north point arrow to be placed in the top right hand corner of the drawing space.

Contours

Contours do not form part of the Site Layout Plan.

Line weight

Line weight/width is extremely important and features such as the services shall be drawn with lines that are more prominent.  The following line weights (mm) can be used:

- 0.10 - 0.15 - 0.25 - 0.30 - 0.35 - 0.50 - 0.70 - 1.00

Line type/Style

The following typical standard line types that can be used:

Hatching

Hatching are angled line patterns to indicate the position of permanent structures.  The spacing between lines shall be consistent at 45º to the structure.  Park Homes must be shown on the plan, but without hatching.

Surfaced areas

Surfaced roads should be indicated by two solid lines as well as paved areas.

Two long broken lines shall be used to indicate gravel roads.

Non standard line types

The following lines could be used for the various services, but must be identified in the Legend as a non standard line type:

Lettering and font styles

Use the standard font style and font size for engineering drawings and do not use stylized fonts.

Create all text in upper case letters, except for certain unit designations such as km, m, mm, kVA, etc.

Key layout

When the port of entry is too large for one sheet, divide the plan into logical sections.  Add a key layout in the title block showing how the various sheets should be joined together to obtain a layout of the entire site.  This key layout shall form part of each sheet.

Facilities

The name of the facility shall be written inside or adjacent to the facility.  If the space is limited, a reference number of the facility, which refers to a description of the facility, is inserted in a table format in or close to the title block.

Fences and gates

Show the position of the security fence and all other fences as well as gates.  Include the height of all fence lines.

Destinations

The destination to the nearest town with a pointing arrow shall appear on all incoming and outgoing roads.

Services

The position of the services is extremely important for future reference and shall be indicated by lines that are more prominent/thicker.  The description of the line types for the various services must be given in the Legend - see non standard lines types above.

The following services, where applicable, must be shown on the Site Layout Pan for future reference:

Water reticulation system

Show the position of the water reticulation system and include the following:

Pipe lines, pipe sizes, type of pipes, valves, meters, boreholes and tanks (include capacities).  Show the direction of flow.

Sewerage network

Show the layout of the sewerage network and include the following:

Pipe lines, pipe sizes, type of pipes, manholes, rodding eyes, septic tanks (include capacities), french drains (include volumes).  Show the invert levels of all manholes as well as the position and level of the bench mark.

Electrical power

Indicate the position of electrical power lines, cables, substations, kiosks, flood lights along the perimeter as well as street lights and area lighting.

Air-conditioning units shall be numbered and listed in table format including the type and size.

Give the source(s) of electrical power.

Telephone lines

Show the position of overhead telephone lines.

Stormwater system

Show the layout of the stormwater system, culverts and sizes as well as inlet and outlet structures.  Give the invert levels of all structures as well as the position and level of the bench mark.

Fire fighting equipment

Include the pump installation, tank and capacity, fire hydrants, valves, meters, fire extinguishers and fire hose reels.

Fire extinguishers should be numbered and listed in table format including the type and size.

Electronic format

A complete set of electronic files shall be placed on CD(s) in a Data Exchange Format (DXF) or DWG format.

Affix a stick-on label to the CD with the following information:

Department of Public Works and logo

Name of port of entry

WCS number

Description:  SITE LAYOUT PLAN

Drawing number(s)

Date issued

Electronic format:  DXF or DWG

Submission

The Consultant must submit A1 and A3 paper prints as well as a CD(s) of the Site Layout Plan(s) to the external Project Manager before the Final Approval Certificate is signed.

The CD(s) must include all the overlays/layering scheme and a compound drawing which includes all the services and information on one Site Layout Plan in DXF/DWG format.

During the repair and maintenance phase, the external Project Manager will forward a request from time-to-time to the Consultants to prepare an A3 print(s) of the Site Layout Plan(s), which will be submitted as part of a report to Department of Public Works.

| FINAL ACCOUNT GUIDELINE |  |  |  |
| --- | --- | --- | --- |
| - SCOPE The purpose of the Final Account is to provide a document to the Department of Public Works to close the project as soon as possible in terms of all expenditure related to the Contractor and Consultant. The Final Account must be compiled correctly, embodied the correct detail, financial information and time information are concerned. This is an audited document, which speaks on behalf of the Consultant. - FILE TYPE AND FORMAT Use “SuperRib” or equivalent quality file. Place Cover Sheet in front and side of the file for reference purposes. Indicate how many files there are, e.g. FILE 1 of 2. Provide plastic file dividers from 1 to 10 as well as sub-dividers according to “Engineer’s RAMP Final Account Checklist”. - GUIDELINE STRUCTURE This Guideline follows the same structure as the “Engineer’s RAMP Final Account Checklist”. Place the “Engineer’s RAMP Final Account Checklis” in front of File 1. The Engineer to complete the “Engineer’s RAMP Final Account Checklist” and signs on second page. The Checklist serves as an index to the Final Account. |  |  |  |
| SEC-TION No. | DESCRIPTION | EXPLANATORY NOTE | COMMENT |
| 1 | FINAL ACCOUNT COVER PAGE | - Standard Cover Page for RAMP |  |
|  |  | - Tender amount: VAT included |  |
|  |  | - Final value of work done: VAT and CPA included |  |
|  |  | - Time information (period is 36 months) |  |
|  |  | - Signature of Consultant |  |
| 2 | FINAL PAYMENT FORMS |  |  |
| 2.1 | Final Statement form | 2.1 Form PRM 044: Final Statement (signed by Consultant) |  |
| 2.2 | Final Payment form | 2.2 Form PRM 044: Final Payment (signed by Consultant). Pro forma attached. Electronic format is available from the external project manager |  |
| 2.3 | WCS final payment printouts | 2.3 Will be provided by DPW on request: |  |
|  |  | - WJ05: Authorised amount (current issue) |  |
|  |  | - WG04: Contract details |  |
|  |  | - WG09: Payments up to date |  |
| 2.4 | Final Progress Payment form | 2.4 Form PW 1526: Provided by external project manager: |  |
|  |  | Final Progress Payment form: Provided by Consultant (signed by Contractor and Consultant) |  |
|  |  | - Progress, Final Cost and Cash Flow calculation and Summary of Work Done |  |
|  |  | - Original Tax invoice |  |
|  |  | - CPA calculation |  |
|  |  | - Penalty: Insert Calculation and Clause |  |
|  |  | - CPG (if applicable). See Letter of Acceptance |  |
|  |  | NOTE: The final amount paid to the Contractor will be the sum of the final certified value of work done plus CPA , minus penalties (if any), minus CPG (if applicable) and plus VAT. |  |

| SEC-TION No. | DESCRIPTION | EXPLANATORY NOTE |  |  |  | COMMENT |
| --- | --- | --- | --- | --- | --- | --- |
| 3 | ORIGINAL SCHEDULE | 3.1 Copy of the Original Schedule: |  |  |  |  |
|  |  | - A complete copy of the Original Schedule of Quantities with hand-written rates must be included - Each page of the document must be initialed and the signature of the tenderer must appear on the last page - If this document is omitted or incomplete or not initialed, a copy thereof must be obtained from the Department of Public Works (DPW) (Contract Section) |  |  |  |  |
| 4 | COMPLETED / FINAL RE-MEASURED SCHEDULE | 4.1 Format |  |  |  |  |
|  |  | A copy of the Complete Schedule (showing final re-measurements) must be provided. The format and layout of this document must correspond with the Original Schedule of Quantities (see Section 3) in respect of the following: |  |  |  |  |
|  |  | Page numbers |  |  |  |  |
|  |  | Heading |  |  |  |  |
|  |  | Schedule number |  |  |  |  |
|  |  | Columns: |  |  |  |  |
|  |  | - Payment refers to |  |  |  |  |
|  |  | - Item numbers |  |  |  |  |
|  |  | - Description of item |  |  |  |  |
|  |  | - Tender: Unit |  |  |  |  |
|  |  | - Tender: Quantity |  |  |  |  |
|  |  | - Tender: Rate |  |  |  |  |
|  |  | - Tender: Amount |  |  |  |  |
|  |  | - Final: Quantity |  |  |  |  |
|  |  | - Final: Amount |  |  |  |  |
|  |  | Totals per page |  |  |  |  |
|  |  | Sub-totals |  |  |  |  |
|  |  | Repair: Summary sheet |  |  |  |  |
|  |  | Maintenance: Summary sheet |  |  |  |  |
|  |  | Repair and maintenance: Summary sheet including Variation Orders, example: |  |  |  |  |
|  |  | Calculation of Tender Sum |  | Tender Amount | Final Amount |  |
|  |  | Total of repair work |  |  |  |  |
|  |  | Total of maintenance work |  |  |  |  |
|  |  | Variation Orders (non scheduled items) |  |  |  |  |
|  |  | VO 1 |  |  |  |  |
|  |  | VO 2 |  |  |  |  |
|  |  | Subtotal and VAT |  |  |  |  |
|  |  | TOTAL: |  |  |  |  |
| SEC-TION No. | DESCRIPTION |  | EXPLANATORY NOTE |  |  | COMMENT |
|  | COMPLETED / FINAL RE-MEASURED SCHEDULE (continued) |  | NOTE: - Rates must be the rates tendered in the Original Schedule of Quantities (Section 3) |  |  |  |
|  |  |  | - Where “SUM” is given as the rate, quantities are not applicable |  |  |  |
|  |  |  | - The Maintenance Points can only be allocated for the time actually on site |  |  |  |
|  |  |  | - Calculations based on points must be checked against the points scored for time actually on site (10 points per month) and calculated from the day of site handover to date of Practical Completion |  |  |  |
|  |  |  | 4.2 Additional documentation to be included: |  |  |  |
|  |  |  | - Housing for Engineer : Include invoices |  |  |  |
|  |  |  | - Additional Tests required by engineer: Include invoices |  |  |  |
|  |  |  | - Day Work: Include duly approved sheets |  |  |  |
|  |  |  | - Call Centre: Include invoices |  |  |  |
|  |  |  | - Provisional sums: Invoices/Variation Orders |  |  |  |
| 5 | FINANCIAL REPORT |  | 5.1 RAMP Annexure 5A/1: Final Total Budget Summary: |  |  |  |
|  |  |  | The amounts on the Final Account Cover Page (item 1), Financial Statement (item 2.1) and Final Payment (item 2.2) must correspond with the detail of Section 3 of this form |  |  |  |
|  |  |  | 5.2 Summary of Variation Orders: |  |  |  |
|  |  |  | - Reveal only non-scheduled items and their re-measured amounts |  |  |  |
|  |  |  | - Include the quantities and amounts for Variation Orders (scheduled items) against scheduled rates in re-measured document at the correct item |  |  |  |
|  |  |  | 5.3 Summary of Work Done: |  |  |  |
|  |  |  | - Include a break down of Variation Orders for non-scheduled rates |  |  |  |
|  |  |  | 5.4 CPA calculation |  |  |  |
|  |  |  | 5.5 Report: Progress, estimated cost and cash flow |  |  |  |
| 6 | ADDITIONAL FUNDS |  | 6.1 Form PRM039/1: Include copies of approved Application for Additional Funds |  |  |  |
| 7 | VARIATION ORDERS (VO’s) |  |  |  |  |  |
| 7.1 | Summary of Variation Orders |  | 7.1 Summary of VO’s for non-scheduled rates |  |  |  |
| 7.2 | VO forms |  | 7.2 Form: PRM033: VO motivation signed and approved by DPW |  |  |  |
|  |  |  | Form PRM037: Variation Order signed and approved by DPW |  |  |  |
|  |  |  | - Include Base Month |  |  |  |
|  |  |  | PRM039/1: Application for Additional Funds (approved) |  |  |  |

| SEC-TION No. | DESCRIPTION |  | EXPLANATORY NOTE | COMMENT |
| --- | --- | --- | --- | --- |
| 7.3 | Application to exceed 5% non-scheduled |  | 7.3 Form PRM 039/1 to 5: |  |
|  |  |  | - Include copies of approved BID-committee application to exceed 5% for non-scheduled items |  |
| 8 | CPA AND INTERIM PAYMENTS |  |  |  |
| 8.1 | CPA calculation sheet |  | 8.1 Calculation of final/total CPA: Include final indices: |  |
|  |  |  | - Base Month must be included |  |
|  |  |  | - CPA calculated on tender amount excluding PC Sums and Provisions Sums |  |
| 8.2 | PROGRESS PAYMENT |  | 8.2 Form PW1526: Include all interim Progress Payments |  |
| 9 | APP: CONTRACT PARTICIPATION GOAL (CPG) |  | 9.0 PA-16.1 (EC): Preference points claim form and affidavit |  |
| 9.1 | Statement |  | 9.1 Documentation: |  |
|  |  |  | - Provide audited proof of points scored: See Letter of Acceptance for percentage (%) to be scored |  |
|  |  |  | - Provide documentation or calculation of penalties applicable |  |
|  |  |  | - Include letter from Auditor to confirm that Contractor has ABE status |  |
| 10 |  | GENERAL CONTRACT INFORMATION |  |  |
| 10.1 |  | DPW Letter of Acceptance | 10.1 Copy of the Letter of Acceptance for appointing of Contractor |  |
| 10.2 |  | Tender Adjudication | 10.2 PA-12: Approval by BID Adjudication Committee PA-21 (EC): Tender evaluation report to the BID Adjudication Committee PA-20: Scoring model |  |
| 10.3 |  | Certificate of Practical Completion | 10.3 Form RAMP Annexure 10.A: Signed by Consultant, Contractor and User Department for all installations |  |
|  |  |  | Approved certificate of Practical Completion with effective date must be provided |  |
| 10.4 |  | Certificate of Completion | 10.4 Form RAMP Annexure 10.B: Submit original Completion Certificate with cover letter to external project manager and file copies of the certificate and cover letter under 10.4 |  |
| 10.5 |  | Termination of Guarantee | 10.5 Form RAMP Annexure 10.D and relevant documentation |  |
| 10.6 |  | Final Approval Certificate | 10.6: Form RAMP Annexure 10.C: Submit original Final Approval Certificate with cover letter to external project manager and file copies of the certificate and cover letter under 10.6 |  |
| 10.7 |  | Certificate of Compliance and Indemnity | 10.7 Form PRM 045: |  |
|  |  |  | - Item 1: Update first paragraph |  |
|  |  |  | - Consultant must sign and date this form. Indicate name of firm |  |
| SEC-TION No. | DESCRIPTION |  | EXPLANATORY NOTE | COMMENT |
| 10.8 | Contract Completion Report |  | 10.8 Form PRM 043 (per contract): |  |
|  |  |  | - Complete all the relevant information and sign document |  |
|  |  |  | - Application of Extension of Time must be included |  |
|  |  |  | NOTE: - Approved Time Extensions (item A10) must be provided as well as Penalties (item A15) imposed. For multi-installation, this information must also be provided for each installation separately. Apply this rule with completion of Sections B and C |  |
|  |  |  | - Penalties: If an amount is inserted, documentation to substantiate the amount should be filed in sub-section 10.8. Ensure that the relevant documents have been signed, dated and approved. |  |
| 10.9 | Original Guarantees |  | 10.9 Confirmation that original Guarantees have been provided to DPW |  |
|  |  |  | Provide an appropriate note if not applicable |  |
| 10.10 | Certificate of Compliance |  | 10.10 Confirmation that original Certificate of Compliance or Boiler Test Certificate (Boiler Inspection Report) (Certificate of Continuance) or both have been provided to DPW. This is compulsory to all electrical or mechanical work |  |
|  |  |  | Provide an appropriate note if not applicable |  |
| 10.11 | Operation and Maintenance Manuals |  | 10.11 Confirmation that Operating and Maintenance Manuals have been approved by the engineer and copies circulated to all parties. A copy must also be submitted |  |
|  |  |  | Provide an appropriate note if not applicable |  |
| 10.12 | “As-Built” drawings |  | 10.12 Confirmation that “As-Built” drawings have been completed and signed by the engineer and submitted to DPW. A copy must also be submitted to the external project manager |  |
|  |  |  | Provide an appropriate note if not applicable |  |
| 10.13 | Site Instruction Books |  | 10.13 Confirmation that Site Instruction Books have been returned to DPW |  |
|  |  |  | Provide an appropriate note if not applicable. |  |

PRM044civ/1

PRM044:  FINALISATION OF FINAL ACCOUNT AND CHECK LIST

PRM044:  Finalisation of Final Account and Check List

1.   Prerequisites for Departmental approval of the final account:

| ACTION |  | YES/NO |
| --- | --- | --- |
| - “Certificate of Completion” (PRM041/3civ) issued. |  |  |
| - “Certificate of Compliance and Indemnity by Consultants” (part 1 only) (PRM045civ) signed. |  |  |
| - “Final Statement” (PRM044/2civ) signed by all consultants involved. |  |  |
| - “Contract Completion Report” (PRM043civ) finalized (not necessary if nil report). |  |  |
| - “Geographical Targeted Procurement Management System” (GTPMS) compliance evidence handed in (relevant DF form). |  |  |
| - Guarantees, certificates, maintenance contracts and manuals |  |  |
| 1.6.1 | When applicable submit guarantees e.g. roof, waterproofing, etc. |  |
| 1.6.2 | Control and confirm that electrical certificate of compliance has been issued. |  |
| 1.6.3 | Control and confirm that maintenance contracts are in place where this is contractually required (Ceded to the Department if required from a nominated sub-contractor). |  |
| 1.6.4 | Where applicable, submit operational and maintenance manuals and maintenance specifications. |  |
| 1.6.5 | When applicable, submit "as-built" drawings for mechanical installations provided by the contractor. |  |
| 1.6.6 | Control and confirm that the original site instruction book(s) and site visitor book have been submitted to the Department. |  |

- Prerequisites for final payment to the contractor:

| ACTION | YES/NO |
| --- | --- |
| - Accepting the final account after approval by the Department in signing of the “Final Statement” by all parties including the contractor (resolution of signature from contractor required). The exception is that the final account shall be deemed signed by the contractor after three months if the contractor fails to object and sign subject to Clause 49.10 of the General Conditions of Contract 2004. Note: The Final Statement can only be presented to the contractor for acceptance subsequent to the acceptance thereof by the Departmental project manager and the Director-General of the Department of Public Works. |  |
| - “Final Approval Certificates” issued (PRM046/2civ). |  |

- Prerequisite for final payment to consultants:

| ACTION | YES/NO |
| --- | --- |
| - Signing of the “Final Statement” by the concerned consultant(s). |  |
| - Signing of the “Certificate of Compliance and Indemnity by Consultants” (PRM045civ Part 1 and 2) by the concerned consultant(s). |  |
| - Submission of “as-built” drawings on A size 0.08mm thickness polyester film marked “as-built” dated and signed by concerned consultant (principal). |  |
| - “Final Approval Certificates” (PRM046/2civ) issued. |  |
| - Compliance certified and reconciliation of the prescribed space and cost norms submitted when applicable, by the concerned consultant(s) (PRM019/1 and -/2). |  |

PRM044civ/2

PRM044:  FINAL STATEMENT

PRM044:  Final Statement

|  |  |  |  |  |  |  |  |  |  |  | WCS No: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  |  |  |  | REFERENCE No: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Project: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PROJECT MANAGER: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CONSULTANT: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Contractor: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| NET AMOUNT OF CONTRACT (EXCLUDING VAT) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | R |  |  |  |  |  |
| APPROVED VARIATIONS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | R |  |  |  | - |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | R |  |  |  | + |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | R |  |  |  |  |  |
| PLUS: CONTRACT PRICE ADJUSTMENT |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | R |  |  |  | + |  |
| LESS: CONTRACT PRICE ADJUSTMENT |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | R |  |  |  | - |  |
| COST OF WORKS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | R |  |  |  |  |  |
| (1) LESS: Penalty for Late Completion (PW156 4.2) |  |  |  |  |  |  |  |  |  |  |  |  |  | R |  |  |  |  |  |  |  |  |  |  |  |  |  |
| (2) LESS: Penalty for Non-Compliance with APP (PW156 4.3) |  |  |  |  |  |  |  |  |  |  |  |  |  | R |  |  |  |  |  |  | + | R |  |  |  | - |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | R |  |  |  |  |  |
| VALUE ADDED TAX |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | R |  |  |  | + |  |
| FINAL VALUE OF CONTRACT |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | R |  |  |  |  |  |
| * RETENTION RETAINED (INCLUDING VAT) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | R |  |  |  |  |  |
| CONSULTING ENGINEER |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | DEPT. PROJECT MANAGER |  |  |  |  |  |  |  |  |  |  |  |
| DATE: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | DATE: |  |  |  |  |  |  |  |  |  |  |  |
| PRINCIPAL AGENT / CONSULTANT |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | pp DIRECTOR-GENERAL DEPARTMENT OF PUBLIC WORKS |  |  |  |  |  |  |  |  |  |  |
| DATE: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | DATE: |  |  |  |  |  |  |  |  |  |  |  |
| I, the undersigned, duly authorised, accept the above statement as correct.: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CONTRACTOR |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | DATE |  |  |  |  |  |  |  |  |  |  |
| * | Applicable for engineering contracts where 12 months retention period is required and retention amount also included in final value of contract |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| FOR DEPARTMENTAL USE |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Contract close out captured on WCS (WG10PU): |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Name: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | OPW: |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  | Date: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

DEPARTMENT OF PUBLIC WORKS

FINAL PAYMENT

PRM044:  Final Payment

|  |  |  |  |  |  |  |  |  |  |  | WCS No: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  |  |  |  | REFERENCE No: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Project: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Contractor: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| NET AMOUNT OF CONTRACT (EXCLUDING VAT) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| NET SUBMISSION (EXCLUDING VAT) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | - |  |
| NET ADDITION (EXCLUDING VAT) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | + |  |
| PLUS: CONTRACT PRICE ADJUSTMENT |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | + |  |
| LESS: CONTRACT PRICE ADJUSTMENT |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | - |  |
| COST OF WORKS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| (1) LESS: Penalty for Late Completion |  |  |  |  |  |  |  |  |  |  |  |  |  | R |  |  |  |  |  |  | - |  |  |  | - |  |
| (2) LESS: Penalty for Non-Compliance with CPG |  |  |  |  |  |  |  |  |  |  |  |  |  | R |  |  |  |  |  |  | - |  |  |  |  |  |
| VALUE ADDED TAX |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | + |  |
| LESS: CREDIT FOR MATERIALS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | - |  |
| FINAL VALUE OF CONTRACT |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| * RETENTION RETAINED (INCLUDING VAT) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| MINUS PREVIOUS PAYMENTS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | - |  |
| FINAL PAYMENT CERTIFICATE No. ............ |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CONSULTING ENGINEER |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | DEPT. PROJECT MANAGER |  |  |  |  |  |  |  |  |  |  |
| DATE: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | DATE: |  |  |  |  |  |  |  |  |  |  |
| N/A |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PRINCIPAL AGENT |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | pp DIRECTOR-GENERAL DEPARTMENT OF PUBLIC WORKS |  |  |  |  |  |  |  |  |  |
| DATE: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | DATE: |  |  |  |  |  |  |  |  |  |  |
| I, the undersigned, duly authorised, accept the above statement as correct.: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CONTRACTOR |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | DATE |  |  |  |  |  |  |  |  |  |
| * | Applicable for engineering contracts where 12 months retention period is required and retention amount also included in final value of contract |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| FOR DEPARTMENTAL USE |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Contract close out captured on WCS (WG10PU): |  |  |  |  |  |  |  |  | Name: |  |  |  |  |  |  |  |  |  | OPW: |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | Date: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |